Table 133.5.3.6.3 Continuous Operations Routes

Type 3 Winter Event:  1 – 6 in. of snow/frozen precipitation in 24 hours OR a trace to 1/2 in. of ice

| PAVEMENT TEMPERATURE RANGE AND TREND | INITIAL OPERATION |  |  |  | SUBSEQUENT OPERATIONS |  |  |  |  | COMMENTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pavement surface at time of initial operation | Maintenance action | Salt spread rates |  | Maintenance action | Salt spread rates |  |  |  |  |
|  |  |  | Pre-wetted solid salt (lb/ln-mi) | Brine (gal/ln-mi) |  | Pre-wetted solid salt (lb/ln-mi) |  | Brine (gal/ln-mi) |  |  |
|  |  |  |  |  |  | Light snow | Heavier snow | Light snow | Heavier snow |  |
| Above 32ºF, steady or rising | Dry, wet, slush or light snow cover | None, see comments |  |  | None, see comments |  |  |  |  | 1) Monitor pavement temperature closely for drops toward 32ºF and below 2) Treat slick patches if needed with pre-wetted salt at 100 lb/lane-mi or brine 44 gal/ln-mi; plow if needed |
| Above 32ºF, 32ºF or below is imminent; ALSO 20 to 32ºF, remaining in range | Dry | Apply brine or pre-wetted salt | 100 | 44 | Plow as needed; reapply brine or pre-wetted solid salt when needed | 100 | 200 | 44 | 88 | 1) Applications will need to be more frequent at lower temperatures and higher snowfall rates 2) Do not apply brine onto heavy snow accumulation or packed snow 3) After heavier snow periods and during light snow fall, reduce salt rate to 100 lb/lane-mi or 44 gal/ln-mi brine; continue to plow and apply salt as needed |
|  | Wet, slush, or light snow cover | Apply brine or pre-wetted salt | 100 | 44 |  |  |  |  |  |  |
| 10 to 20ºF, remaining in range | Dry, wet, slush, or light snow cover | Apply pre-wetted salt | 200 |  | Plow as needed; reapply pre-wetted solid salt when needed | 200 | 250 |  |  | 1) Reduce salt rate to 200 lb/lane-mi after heavier snow periods and during light snow fall; continue to plow and apply salt as needed 2) Liquid calcium chloride may be used for pre-wetting salt at colder temperatures. |
| Below 10ºF, steady or falling | Dry or light snow cover | Plow as needed |  |  | Plow and apply salt/abrasive mix as needed |  |  |  |  | 1) As pavement becomes slick apply salt/abrasive mix to enhance traction. Salt will have limited melting power at this temperatures |

Notes:  SALT APPLICATIONS.  (1) Time initial and subsequent salt applications to prevent deteriorating conditions or development of packed and bonded snow.  (2) Anticipate increases in snowfall intensity.  Apply higher rate treatments prior to or at the beginning of heavier snowfall periods to prevent development of packed and bonded snow.  (3) Apply salt ahead of traffic rush periods occurring during storm.

PLOWING.  If needed, plow before salt applications so that excess snow, slush, or ice is removed and pavement is wet, slushy, or lightly snow covered when treated.