### Aquatic Glyphosate General Information

Common Name: Glyphosate		Brand name: Rodeo

EPA Number: 62719-324

MoDOT Commodity Code: 6759050722

Form: Liquid.

Target: Principal targets are aquatic weeds and brush to be controlled in or near water.

Comments: May be sprayed into or over water. A broad spectrum herbicide which will control plants. Used as a foliar spray on post-emergent plants. Avoid spraying on foliage of desirable plants.

### Safety

- Personal Safety

  - Read the label thoroughly.

  - Wear long sleeved shirt, long pants, impermeable gloves, and adequate eye protection when spraying.

  - Wear apron, full face shield and gloves while mixing.

  - Avoid breathing spray mist or dust from mixing.

  - Thoroughly wash with detergent and water, and wash all outer clothing after each use.

  - Do not use in galvanized or unlined steel container.

- Environmental Concerns

  - Read the label thoroughly.

  - Rodeo is water soluble and nonselective. Take precautions to prevent damage to desirable plants through spray drift.

  - Dispose of excess mixture and all rinse water in accordance with hazardous waste disposal regulations.

  - Triple rinse containers, puncture, and dispose of in a sanitary landfill.

### Mixing Instructions

For handgun spraying, mix 1 – 1.5 gallon per 100 gallons of water. Use approved aquatic adjuvants at the recommended rate.

### Application Instructions

- For best results apply as a foliar spray when target plants are fully emergent and actively growing.

- Spray until the foliage is thoroughly wet, but avoid excessive runoff.

- Take precautions to protect desirable plants from spray drift or runoff.

Go to http://www.cdms.net/LabelsMsds/LMDefault.aspx for label information