### 2,4-D General Information

Common Name: 2,4-D	Brand Name: DMA 4 IVM

EPA Number: 62719-3

MoDOT Commodity Code: HERB50871

Form: Liquid.

Target:  Principle target is broadleaf weeds.

Comments: A selective herbicide; controls broadleaf weeds without harming most grasses. 2,4-D is sold under a variety of brand names.  MoDOT switched to amine formulation in 2011 with an aquatic label.

### Safety

- Personal Safety

  - Read the label thoroughly.

  - Wear long sleeved shirt, long pants, impermeable gloves, and adequate eye protection when spraying.

  - Wear apron, full face shield and gloves while mixing.

  - Avoid breathing spray mist or dust from mixing.

  - Thoroughly wash with detergent and water, and wash all outer clothing after each use.

- Environmental Concerns

  - Read the label thoroughly.

  - Do not apply directly to water.

  - A number of valuable plants including soybeans, grapes, cotton, tobacco, alfalfa, fruits, roses and most garden vegetables are highly susceptible to 2,4-D. Take strong precautions to prevent damage from spray drift. Do not use 2,4-D after these plants have come up and are in an active growing condition.

  - Dispose of excess spray mixture and rinse water from cleaning operations in accordance with hazardous waste disposal regulations.

  - Triple rinse containers, puncture, and dispose of in a sanitary landfill.

  - This product can become volatile and cause damage a mile or more off right of way.

### Mixing Instructions

For broadcast spraying, mix to provide a coverage of 2 quarts per acre. For handgun spraying, mix 2 quarts per 100 gallons of water. See label instructions for best use of each brand name.  Use approved adjuvants at the recommended rate.

### Application Instructions

- Emulsions of oil-soluble herbicides in water require continuous agitation.

- Take strong precautions to prevent contact with desirable plants through spray drift or volatility.

- See label instructions for best use of each brand name.

- Do not use within a mile of susceptible plants.

Go to http://www.cdms.net/LabelsMsds/LMDefault.aspx for label information