GROOVING FOR PAVEMENT MARKING

1.0  Description.  This work shall consist of furnishing and installing a groove in the pavement for placement of pavement markings as shown on the plans or as directed by the engineer.

2.0   Construction Requirements.

2.1  The grooves shall be cut such that the surface of the groove is uniform with minimal variation in height.

2.2  The grooves shall be located where the final pavement marking will be placed according to the plans or as directed by the engineer.

2.3  On bituminous pavements the width of the groove shall be 7 inches.  On PCCP pavements the width of the groove shall be 10 inches.

2.4  The final depth of the groove on bituminous pavements shall be 50 mils, plus or minus 5 mils.  The final depth of the grove on PCCP pavement shall be 80 mils, plus or minus 5 mils.

2.5  The groove shall be clean and dry before the installation of the pavement marking can begin.

2.6  All debris resulting from the installation of the grooves shall be removed and disposed of by the contractor.

3.0  Method of Measurement.  Final measurement will not be made except for authorized changes during construction or where appreciable errors are found in the contract quantity.  Where required, grooves will be measured separately and made to the nearest 1/10 station.  The revision or correction will be computed and added to or deducted from the contract quantity.

4.0  Basis of Payment.  The accepted quantity of grooves will be paid for at the contract unit price per each of the pay items included in the contract.  Payment will be considered full compensation for all labor, equipment, and material necessary to complete the described work, including loading, hauling, stockpiling and disposal of material; and any other incidental items.