STANDARD ACRYLIC WATERBORNE TRAFFIC MARKING PAINT

1.0  Description.  This work shall consist of furnishing and the installation of pavement markings using standard acrylic waterborne traffic marking paint.  The pavement markings shall consist of a waterborne acrylic pavement marking paint and MoDOT Type P glass beads  All materials and construction procedures shall meet MoDOT specifications and specifically the applicable requirements listed in Sec 620.40, except as noted herein.

2.0  Material.  All material shall be in accordance with Division 1000, Material Details, and specifically as follows.

| Item | Section |
| --- | --- |
| Type P Drop-On Glass Beads | 1048.40 |
| High Build Acrylic Waterborne Pavement Marking Paint | 1048.90 |

The following shall replace existing section 1048.90.2.2 in its entirety.

1048.90.2.2  Acrylic Emulsion Polymer.  The acrylic emulsion polymer used in the manufacture of the paint shall be Rohm & Haas E-2706, Dow DT 211 or equal.

3.0   Construction Requirements.

3.1  The width of the line shall be as shown on the plans.

3.2  The waterborne paint shall be applied at a minimum thickness of 15 mils.

3.3  The completed pavement marking system shall meet the initial retroreflectivity requirements of Sec. 620.2.4.1.

4.0  Method of Measurement.  Final measurement will not be made except for authorized changes during construction or where appreciable errors are found in the contract quantity.  The revision or correction will be computed and added to or deducted from the contract quantity.

Where required, measurement of 4 inch, 6 inch, 8 inch or 24 inch pavement marking will be made to nearest linear foot.  Where intermittent lines are specified, deductions will be made for the gaps in pavement marking.

5.0  Basis of Payment.  The accepted quantity of acrylic waterborne pavement marking paint will be paid for at the contract unit price for each of the pay items included in the contract.