Table 133.5.3.6.2 Continuous Operations Routes

Type 4 Winter Event:  Dusting to 1 in. of snow, sleet or other frozen precipitation

| PAVEMENT TEMPERATURE RANGE AND TREND | INITIAL OPERATION |  |  |  | SUBSEQUENT OPERATIONS |  |  | COMMENTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pavement surface at time of initial operation | Maintenance action | Salt Spread Rates |  | Maintenance action | Salt Spread Rates |  |  |
|  |  |  | Pre-wetted solid salt (lb/ln-mi) | Brine (gal/ln-mi) |  | Pre-wetted solid salt (lb/ln-mi) | Brine (gal/ln-mi) |  |
| Above 32ºF, steady or rising | Dry, wet, slush or light snow cover | None, see comments |  |  | None, see comments |  |  | 1) Monitor pavement temperature closely for drops toward 32ºF and below 2) Treat icy patches if needed with pre-wetted solid salt at 100 lb/lane-mi; plow if needed |
| Above 32ºF, 32ºF or below is imminent; ALSO 15 to 32ºF, remaining in range | Dry | Apply brine or pre-wetted solid salt | 100 | 44 | Plow as needed; reapply liquid or solid chemical when needed | 100 | 44 | 1) Applications will need to be more frequent at lower temperatures and higher snowfall rates 2) It is not advisable to apply a straight brine at the indicated spread rate when the pavement temperature drops below 20ºF 3) Do not apply brine onto heavy snow accumulation or packed snow |
|  | Wet, slush, or light snow cover | Apply liquid or solid salt | 100 | 44 |  |  |  |  |
| 0 to 15ºF, remaining in range | Dry, wet, slush, or light snow cover | Apply pre-wetted solid chemical | 200 |  | Plow as needed; reapply pre-wetted solid chemical when needed | 200 |  | 1) Abrasives may be added to the salt to enhance traction at colder temperatures 2) Liquid calcium chloride may be used for pre-wetting solid salt at colder temperatures |
| Below 0ºF, steady or falling | Dry or light snow cover | Plow as needed |  |  | Plow and apply salt/abr. mix as needed |  |  | 1 If pavement becomes slick apply salt/abrasive mix to enhance traction. Salt will have limited melting power in this temperature range. 2) Pre-wet salt/abrasive mix with liquid calcium chloride. |

Notes:  SALT APPLICATIONS.  (1) Time initial and subsequent chemical applications to prevent deteriorating conditions or development of packed and bonded snow.  (2) Apply salt ahead of traffic rush periods occurring during storm.

PLOWING.  If needed, plow before salt applications so that excess snow, slush, or ice is removed and pavement is wet, slushy, or lightly snow covered when treated.