Manual on

IDENTIFICATION, ANALYSIS AND CORRECTION

OF HIGH-CRASH LOCATIONS  (the HAL Manual)

Third Edition  -  1999

Prepared for:  	Technology Transfer Assistance Program

Missouri Department of Transportation

Prepared by:  	Department of Civil and Environmental Engineering

University of Missouri-Columbia

The first two editions of this manual used the term “High-Accident Locations” and have often been referred to as the HAL manual.  This edition adopts the term “High-Crash Locations” to reflect a change in terminology over the past few years.

The First Edition was published in 1975 by the Midwest Research Institute.  The work was sponsored by the Missouri State Highway Commission in cooperation with the Missouri Division of Highway Safety and the Federal Highway Administration, U. S. Department of Transportation.  The First Edition was written by Jerry Graham and John Glennon.

The Second Edition (1990) was sponsored through the Technology Transfer Assistance Program (TTAP) under a project with the Civil Engineering Department, University of Missouri-Rolla.  The project director was Charles E. Dare, Professor of Civil Engineering at UMR.

This Third Edition was also sponsored by TTAP through a project with the Civil and Environmental Engineering Department, University of Missouri-Columbia.  The project directors were Profs. Mark R. Virkler and Kristen L. Sanford Bernhardt.  Graduate student Mahdi Shehab and undergraduate students Victoria Goessling, Jarrett Groccia, and Barbara Lappin assisted in the effort.

Appreciation is extended to Mr. Frank Abart, Director, Boone County Public Works Department;  Mr. Steve Ake, Central Missouri State University, Missouri Safety Center;  Mr. Wally Campbell, MoDOT District 5;  Dr. Charles Dare, MoDOT Traffic Division;   Mr. Jim Radmacher, MoDOT Research, Development and Technology Division;  Mr. Mark Schroyer, Federal Highway Administration – Missouri Division;  Mr. Randy Silvey, Missouri Division of Highway Safety; and Mr. Brian Umfleet, MoDOT District 6; all of whom contributed to the revision of this manual.

December 1999

The opinions, findings and conclusions expressed in this publication are not necessarily those of the U. S. Department of Transportation, Federal Highway Administration.  This report does not constitute a standard, specification or regulation.

APPENDIX A

NON-crash-BASED PROCEDURES

In addition to the crash files, procedures, and summaries that have been described in this manual, locations needing improvement can also be identified through other means.  Information other than the numbers, types, and locations of crashes can often help to identify hazardous locations before a large number of crashes occur.  Other sources of information include citizen complaints and suggestions concerning road safety and repair, employee reports of hazardous locations or ideas for improving traffic safety, and road safety audits.  The city should have a system in place to receive and act on this information in a timely and organized manner.

non-crash Sources of information for improvements

Citizen Complaints

Responses to complaints from citizens should be acted upon according to the importance of the situation to public health and well being.  A complaint regarding a hazardous situation could necessitate an immediate response, such as replacing a missing STOP sign.  A telephone call reporting a large pothole, on the other hand, may be justification to alter the street maintenance schedule.

Employee Reports

All city employees and officials, not just police officers, should be encouraged to submit ideas for improving traffic safety.  Files on public and employee input should include:

- The time and date when the information was received,

- The nature of the reported hazard,

- The name of the person who was assigned the responsibility to investigate the problem,

- The actions taken to remedy the situation, and

- The time and date when the corrective action was completed.

Road Safety Audits

The road safety audit is a relatively new technique aimed at identifying potential road hazards on existing and future roads.  In a report published by the Institute of Transportation Engineers (ITE), a road safety audit is defined as, “a formal examination of an existing or future road or traffic project, or any project that interacts with road users, in which an independent, qualified examiner looks at the project’s crash potential and safety performance.”  Two of the key aspects of a road safety audit are that it is a formal, unbiased evaluation of the roadway (primarily identifying safety problems) and that it employs qualified and experienced auditors.

A road safety audit has two main objectives.  The first objective is to identify areas on roads where potential crashes may occur.  The second objective is to reduce or eliminate safety problems by taking proper remedial measures.  Benefits realized by safety audits include:

- Reduction in the frequency and severity of traffic crashes,

- Elimination of post-construction work,

- Increase in the economic benefits of a project by reducing the lifecycle costs of a project, and

- Promotion of safe design practices during planning, design, construction, and maintenance stages of projects.

Application of Road Safety Audits

Road safety audits can be conducted at different stages of the project, including:

- Feasibility Stage:  Road safety audits can affect the scope of the project, selection of routes, design standards, the road network currently in service, and many of the other activities taking place at this stage.

- Preliminary Design Stage: Aspects of the project that can be affected by safety audits during this stage include horizontal and vertical alignment, lane width, shoulder width, intersection layouts, and super-elevation.

- Detailed Design Stage:  During this stage, many aspects of the detailed design are considered, such as line markings, signs, delineation, lighting, and details of intersection layouts.

- Pre-opening Stage:  The auditor or audit team should drive, ride, and walk through the facility at different times and under different weather and climate conditions to locate areas where the user is at risk.

- In-service Stage:  During this stage, a systematic examination of the existing roads is performed to evaluate their safety.  This type of audit can be used to monitor a newly opened facility or to evaluate the safety of an existing road or network of existing roads.

Conducting Road Safety Audits

The road safety audit is a valuable tool for preventing crashes.  It can be performed with a limited amount of crash and traffic data, which makes it especially feasible and cost-effective for small cities.  Sample worksheets for safety audits are provided in Figures A-1 and A-2.  The audit is made up of the following steps:

- Select an auditor or audit team:  The auditor or audit team should be experienced in the field of traffic safety and management, crash investigation, road design, and human factor analysis.  The selection should be such that the auditor or audit team will conduct the audit in an independent and objective manner.  Independence of the auditor or audit team can be ensured by hiring qualified consultants in the city or by utilizing an auditor from another city.

- Conduct the road safety audit:  A program should be developed that ensures the auditing of the entire network of roads and streets.  Then, an audit checklist should be formulated covering all the important safety problems.  This checklist is used as a supplement to support the experience and knowledge of the auditor or audit team.  During the audit, safety must be considered from the viewpoint of all road users, and all possible movements of traffic must be examined.  The audit should also address different climate conditions, conditions at different times of the day, and different traffic conditions.  Finally, the audit should address the possibility of enhancing safety by providing a more consistent street environment.

- Produce a road safety report:  The final report describes the results, and hence the safety needs for the street network.  Priorities and general auditor recommendations may be included in the report.

- Hold a follow-up evaluation:  The auditor or audit team, persons with jurisdiction over the network, and those funding the project should discuss the results and findings of the audit in a follow-up meeting.  During the meeting, some safety needs are given priorities over others.  Any action regarding the audit itself should be documented, as well as resulting programs, schedules, and safety actions to be taken.

Setting up a system for receiving information

The city should have a well-organized system for receiving information from individuals, prioritizing city responses, assigning work to be done, and documenting job completion.  This allows the city not only to respond to citizen complaints more effectively, but also to expand the ability of the city to detect traffic safety problems throughout the entire jurisdiction.  The system can be set up as follows:

- Establish a specific contact point in the city offices to receive all complaints and suggestions concerning local traffic safety.  Each contact must be logged into a permanent record giving the name, address, and phone number of the individual making the report, the time the report was received, and a description of the problem reported.

Safety Audit Checklists for Existing Streets

Auditor(s):  _______________________________________	Date:  _________________

Location (Reference Map included):

TRAFFIC SIGNS

Traffic signs must: 1) Fulfill a need, 2) Command attention, 3) Convey a clear, simple message, 4) Command respect of road users, and 5) Give adequate time for proper response.  When correcting problems, priority is recommended for regulatory signs (i.e. Stop, Yield, Speed Limit, Do Not Enter, and Road Closed) and for major warning signs (i.e. Stop Ahead, Yield Ahead, Turn, Curve, and Railroad Crossings).

Check

Are signs visible, both day and night, at a distance that provides response time for motorists?

Is sign visibility affected by:

	Vegetation, Dirt, Other Materials?

	Sharp Curves?

	Steep Hills?

	Other Signs?

	Poor Lighting?

	Reflectivity at Night?

Have damaged, vandalized, or missing signs been repaired or replaced?

Does the sign have a clear and simple message?

Are signing practices consistent at similar locations?

Are signs correctly positioned with respect to:

	Lateral Clearance?  (2 feet recommended)

	Height?  (7 feet to bottom of the sign recommended)

Are sign supports breakaway or yielding?

	If not, are the sign supports located to minimize exposure to traffic?

Site-specific factors may require engineering judgment.  The Manual on Uniform Traffic Control Devices (MUTCD) is the basis for all traffic control device standards.  The MUTCD and applicable state and local standards should be referenced as needed.  The necessary advance warning distance depends on several factors such as vehicle speed, site conditions, and required motorist action; consult the MUTCD for further guidance.

FIGURE A-1  Audit Checklist for Traffic Signs On Existing Streets (From Haiar and wilson 1999)

Safety Audit Checklists for Existing Streets

Auditor(s):  _______________________________________	Date:  _________________

Location (Reference Map included):

INTERSECTIONS

Site-specific factors often require engineering judgment.  The Manual on Uniform Traffic Control Devices (MUTCD) and applicable state and local standards should be referenced as needed for guidance as to the appropriate traffic control and sight distance for an intersection.  The signing checklist provides a more detailed examination of signing issues.

Check

Is the visibility of the intersection or any approaches limited by:

	Parked or Queued Traffic?

	Signs, Utility Poles, Fences?

	Embankments?

	Buildings?

	Vegetation?

	Other Sight Obstructions?

Has an effort been made to improve the sight distance of the intersection before installing traffic control measures?

	An engineering study is usually necessary for the placement of traffic control.

	Use of stop signs is not recommended for speed control.

Are hidden or unexpected intersections located on:

	Hills or curves?

	At the end of high-speed streets?

	Streets that do not intersect at 90?

If so, additional warning for the motorist may be necessary.

Are pedestrians (children, bicyclists, etc.) and motorists readily visible at the intersection?

FIGURE a-2:  Audit Checklists for Street Intersections (from haiar and wilson 1999)

- Enact a procedure to prioritize complaints in the event there are multiple complaints received at about the same time.  The following four-level priority system is suggested:

Priority A:	URGENT.  Should respond as soon as possible (day, night, weekends, or holidays), suspending lower priority work if necessary.  This condition represents an immediate hazard to the public, such as roadside fixture knockdown onto street, traffic signal bulb out, or stop sign missing.

Priority B:	MODERATE RISK.  Should respond as soon as possible, but within normal working hours and only after Priority A repairs are finished.  This situation results in some danger to the motoring public and most drivers would normally not expect it to exist.  Examples include roadside fixture knockdown onto shoulder, warning sign missing, or sight distance restricted due to vegetation.

Priority C:	LOW RISK.  Only slight danger to motoring public if some degree of caution is not exercised.  Repair should be accomplished with more urgency than routine maintenance.  Examples are:  lighting fixture malfunction, lack of pavement stripe, or loose gravel on a paved surface.

Priority D:	ROUTINE MAINTENANCE.  Repair not urgent, situation is a reasonably common occurrence, with little or no hazard to the motoring public.  Repair would be considered as routine maintenance, but maintenance schedule could be altered to give earlier attention to reported condition.  Examples are spalled pavement areas or small potholes.

- Any corrective action should be recorded on a form designed to describe the complaint, its location, the priority of action to be taken, the name of the person assigned to investigate and handle the problem, the time the repair work was initiated, the nature of the work that was completed, and the time when the work was completed.

- After the work has been completed, the person who filed the complaint or provided the suggestion should be contacted to inform him or her of the actions taken.  Then, a permanent record should be kept, by location, to supplement the high-crash location countermeasure selection process.

It is also advisable for the city engineer to record other data that will prove useful for traffic studies, city planning, and activity reports.  Examples of supplementary information that should be kept include:

- Dates and descriptions of major street and intersection improvements,

- Dates of completion and descriptions of any new, major facility that causes changes in traffic volumes or traffic patterns, and

- Files on public input and city employee reports.

REFERENCES

Haiar, K. A. and E. M. Wilson, “Adapting Safety Audits for Small Cities.”  Preprint, 78th Annual Meeting of the Transportation Research Board, January 1999.

ITE Technical Committee 4S-7, “Road Safety Audit:  A New Tool for Crash Prevention.”  ITE Journal, February 1995, pp. 15-22.

“Local Highway Safety Improvement Program – Users’ Guide,” Federal Highway Administration, July 1986.

“Local Highway Safety Studies – Users’ Guide,” Federal Highway Administration, July 1986.

APPENDIX B

PROBABLE CAUSES FOR CRASH PATTERNS AND GENERAL COUNTERMEASURES

The primary purpose of the crash pattern-cause-countermeasure table (Table B-1) is to assist the user in establishing a list of general countermeasures (or possible improvements) for a high-crash location.  It is assumed that particular crash patterns have associated probable causes.  Crash patterns are identified from crash summaries and collision diagrams.  Probable causes relating to crash patterns are inferred from crash reports, on-site reviews, and other traffic studies conducted at the site.

Table B-1 is a basic guide to the general types of countermeasures that have been found to be effective in crash reduction.  There may be other improvements not in the table that could be appropriate for a particular high-crash location.  Those improvements may be identified by professional judgment or by consulting with other engineers.

The crash pattern-cause-countermeasure table is organized according to the following crash patterns:

- Right-angle collisions at un-signalized intersections

- Right-angle collisions at signalized intersections

- Rear-end collisions at un-signalized intersections

- Rear-end collisions at signalized intersections

- Pedestrian crashes at intersections

- Pedestrian crashes at locations between intersections

- Fixed object collisions

- Fixed object collisions and/or vehicles running off road

- Collisions with parked vehicles or vehicles being parked

- Collisions at driveways

- Wet pavement crashes

- Crashes at night

- Collisions at railroad grade crossings

- Sideswipe or head-on collisions between vehicles traveling in opposite directions

- Lane change, sideswipe, or turning collisions between vehicles traveling in the same direction

- Left-turn collisions at intersections

- Right-turn collisions at intersections

- Pedestrian crashes at intersections

REFERENCES

Box, P., “Accident Pattern Evaluation and Countermeasures,” Traffic Engineering, pp. 38-43, August 1976.

“Highway Safety Engineering Studies – Procedural Guide,” Federal Highway Administration, Report No. FHWA-TS-81-220, November 1981.

“Local Highway Safety Studies – User’s Guide,” Federal Highway Administration, July 1986.

“Manual of Transportation Engineering Studies,” 1st Edition, Institute of Transportation Engineers, Prentice-Hall, Englewood Cliffs, NJ, 1994.

| CRASH PATTERN | PROBABLE CAUSE | COUNTERMEASURE |
| --- | --- | --- |
| Right-angle collisions at | restricted sight distance | 1-remove sight obstructions |
| un-signalized |  | 2-restrict parking near corners |
| intersections |  | 3-install warning signs * |
|  |  | 4-install yield signs * |
|  |  | 5-install stop signs * |
|  |  | 6-install overhead flashing beacon * |
|  |  | 7-channelize intersection |
|  |  | 8-install/improve street lights at intersection |
|  |  | 9-install traffic signals * |
|  |  | 10-set appropriate speed limit ** |
|  |  | 11-improve intersection approach angle |
|  | high approach speed | 1-set appropriate speed limit ** |
|  |  | 2-install rumble strips |
|  |  | 3-install overhead flashing beacon * |
|  | large total traffic volume at | 1-install stop signs * |
|  | location | 2-restrict parking near corners |
|  |  | 3-add traffic lanes |
|  |  | 4-re-route through-traffic |
|  |  | 5-install signals * |
|  | inadequate roadway lighting | install/improve street lights at intersection |
|  | inadequate advance | install/improve warning signs * |
|  | intersection warning signs |  |
|  | inadequate traffic control | 1-upgrade traffic control devices |
|  | devices | 2-increase enforcement |
| Right-angle collisions | restricted sight distance | 1-remove sight obstructions |
| at signalized |  | 2-restrict parking near corners |
| intersections |  | 3-install/improve warning signs * |
|  |  | 4-set appropriate speed limit ** |
|  |  | 5-provide adequate channelization |
|  |  | 6-provide pavement markings to supplement signs |

Table b-1:  general countermeasures for crash patterns and their probable causes

| CRASH PATTERN | PROBABLE CAUSE | COUNTERMEASURE |
| --- | --- | --- |
| Right-angle collisions | poor visibility of traffic signals | 1-remove sight obstructions |
| at signalized |  | 2-set appropriate speed limit ** |
| intersections (cont'd) |  | 3-install or improve warning sign(s) * |
|  |  | 4-install 12-inch signal lenses * |
|  |  | 5-install signal visors or back plates |
|  |  | 6-install overhead signals * |
|  |  | 7-add signal heads * |
|  |  | 8-re-locate signals |
|  | inadequate traffic signal | 1-adjust yellow change interval |
|  | timing or type of signal | 2-add all-red clearance interval |
|  |  | 3-adjust phase times and cycle time |
|  |  | 4-install multi-dial controller |
|  |  | 5-install traffic actuated signal |
|  |  | 6-adjust minimum green or extension time |
|  |  | 7-interconnect traffic signals and improve timing |
|  |  | 8-install signal speed signs * |
|  | excessive speed | 1-set appropriate speed limit ** |
|  |  | 2-adjust yellow change interval |
|  |  | 3-install rumble strips |
|  | inadequate roadway lighting | install/improve street lights at intersection |
|  | inadequate advance | install/improve warning sign(s) * |
|  | intersection warning signs |  |
|  | large total intersection volume | 1-add lane |
|  |  | 2-adjust signal timing |
| Rear-end collisions at | pedestrians crossing roadway | 1-improve crosswalk markings and/or signs * |
| un-signalized |  | 2-install/improve street lights at intersection |
| intersections |  | 3-relocate crosswalk |
|  | excessive speed | set appropriate speed limit ** |

Table b-1 (CONT.):  general countermeasures for crash patterns and their probable causes

| CRASH PATTERN | PROBABLE CAUSE | COUNTERMEASURE |
| --- | --- | --- |
| Rear-end collisions at | large volume of vehicles | 1-increase curb radii |
| un-signalized | turning | 2-install turning lanes |
| intersections (cont'd) |  | 3-prohibit turns |
|  | slippery surface | 1-overlay pavement (friction course) |
|  |  | 2-chip and seal or slurry seal approaches |
|  |  | 3-groove pavement surface |
|  |  | 4-provide adequate drainage and/or improve crown |
|  |  | 5-set appropriate speed limit ** |
|  |  | 6-use "SLIPPERY WHEN WET" sign * |
|  | driver not aware of | 1-install/improve warning signs * |
|  | intersection | 2-install overhead flashing beacon * |
|  |  | 3-improve intersection approach angle |
|  | inadequate roadway lighting | install/improve street lights at intersection |
|  | lack of adequate gaps | 1-install traffic signal * |
|  |  | 2-install stop sign * |
| Rear-end collisions at | poor visibility of traffic signals | 1-install/improve warning sign * |
| signalized intersections |  | 2-install 12-inch signal lenses * |
|  |  | 3-install signal visors or back plates |
|  |  | 4-install overhead signals * |
|  |  | 5-add signal heads * |
|  |  | 6-re-locate signals |
|  |  | 7-remove sight obstructions |
|  |  | 8-set appropriate speed limit ** |
|  | inadequate traffic signal | 1-adjust yellow change interval |
|  | timing | 2-add all-red clearance interval |
|  |  | 3-adjust phase times and cycle time |
|  |  | 4-install multi-dial controller |
|  |  | 5-install traffic actuated signal |
|  |  | 6-adjust minimum green or extension time |
|  |  | 7-interconnect traffic signals and improve timing |

Table b-1 (CONT.):  general countermeasures for crash patterns and their probable causes

| CRASH PATTERN | PROBABLE CAUSE | COUNTERMEASURE |
| --- | --- | --- |
| Rear end collisions at | slippery surface | 1-overlay pavement (friction course) |
| signalized intersections |  | 2-chip and seal or slurry seal approaches |
| (cont'd) |  | 3-groove pavement surface |
|  |  | 4-provide adequate drainage and/or improve crown |
|  |  | 5-set appropriate speed limit ** |
|  |  | 6-use "SLIPPERY WHEN WET" sign * |
|  | pedestrians crossing roadway | 1-improve crosswalk markings/signs * |
|  |  | 2-provide pedestrians with "WALK" phases |
|  |  | 3-install/improve street lights at intersection |
|  | unwarranted signals | remove signal * |
|  | large volume of vehicles | 1-prohibit turn |
|  | turning | 2-install turn lane |
|  |  | 3-increase curb radii |
|  |  | 4-add left-turn signal phase |
|  | inadequate roadway lighting | install/improve street lights at intersection |
| Pedestrian crashes | inadequate protection for | 1-add pedestrian refuge island |
| at intersections | pedestrians | 2-install pedestrian barrier |
|  |  | 3-install pedestrian signals * |
|  |  | 4-install pedestrian bridge or tunnel |
|  | inadequate traffic signals | 1-add pedestrian "WALK" phase * |
|  |  | 2-improve timing of pedestrian phase |
|  | excessive speed | 1-install/improve warning sign * |
|  |  | 2-set appropriate speed limit ** |
|  |  | 3-increase enforcement |
|  |  | 4-install pedestrian barrier |
|  | inadequate signal timing | re-time signal |

Table b-1 (CONT.):  general countermeasures for crash patterns and their probable causes

| CRASH PATTERN | PROBABLE CAUSE | COUNTERMEASURE |
| --- | --- | --- |
| Pedestrian crashes | school crossing area | 1-remove parking from crosswalk location |
| at intersections (cont'd) |  | 2-remove sight obstructions |
|  |  | 3-add school zone markings * |
|  |  | 4-install school crossing signs * |
|  |  | 5-install school speed limit signs * |
|  |  | 6-install school crossing signals * |
|  |  | 7-use school crossing guards |
|  |  | 8-revise school route plan map * |
|  |  | 9-install pedestrian bridge or tunnel |
|  | sight distance inadequate | 1-remove sight obstructions |
|  |  | 2-install/improve pedestrian crosswalk |
|  |  | 3-install/improve pedestrian crossing signs * |
|  |  | 4-reroute pedestrian path/mid-block crossing |
|  |  | 5-restrict parking near corner/crosswalk |
|  | inadequate/improper | 1-install thermoplastic markings |
|  | pavement markings | 2-provide signs to supplement markings |
|  |  | 3-improve/install pavement markings |
| Pedestrian crashes at | driver has inadequate warning | 1-install/improve warning signs * |
| locations between | of frequent mid-block | 2-set appropriate speed limit ** |
| intersections | crossings | 3-install pedestrian barrier |
|  |  | 4-prohibit parking |
|  | pedestrians walking on road | 1-install sidewalks |
|  | or jay-walking | 2-install "CROSS ONLY AT CROSSWALK" sign * |
|  |  | 3-install pedestrian barrier |
|  | distance too long to nearest | 1-install additional crosswalks and signs * |
|  | crosswalk | 2-install pedestrian actuated signals * |
|  | excessive speed | 1-install/improve warning sign * |
|  |  | 2-set appropriate speed limit ** |
|  |  | 3-increase enforcement |
|  |  | 4-install pedestrian barrier |

Table b-1 (CONT.):  general countermeasures for crash patterns and their probable causes

| CRASH PATTERN | PROBABLE CAUSE | COUNTERMEASURE |
| --- | --- | --- |
| Pedestrian crashes at | inadequate roadway lighting | improve roadway lighting |
| locations between |  |  |
| intersections (cont'd) |  |  |
|  | lack of adequate gaps | 1-provide traffic signal * |
|  |  | 2-install/improve pedestrian crosswalk traffic |
|  |  | control devices * |
|  |  | 3-provide pedestrian signal * |
|  | inadequate/ improper | 1-install thermoplastic markings |
|  | pavement markings | 2-provide signs to supplement markings |
|  |  | 3-improve/install pavement markings |
| Fixed object collisions | objects located too close to | 1-remove/re-locate large objects |
|  | the roadway | 2-install object marker * |
|  |  | 3-modify poles/posts with breakaway features |
|  |  | 4-eliminate poles by burying utility lines |
|  |  | 5-install barrier curbs or guardrail |
|  |  | 6-install crash cushioning device |
|  | excessive speed | set appropriate speed limit** |
|  | slippery surface | 1-provide adequate drainage |
|  |  | 2-provide "SLIPPERY WHEN WET" signs * |
|  |  | 3-widen lane |
|  |  | 4-improve skid resistance |
|  | inadequate roadway lighting | install/improve roadway lighting |
|  | inadequate/improper | install/improve pavement markings |
|  | pavement markings |  |
|  | inadequate roadway design | 1-install/improve warning signs * |
|  | for conditions | 2-provide proper superelevation |

Table b-1 (CONT.):  general countermeasures for crash patterns and their probable causes

| CRASH PATTERN | PROBABLE CAUSE | COUNTERMEASURE |
| --- | --- | --- |
| Fixed object collisions | inadequate traffic control | paint/install reflectors on obstructions |
| (cont'd) | devices and guardrails |  |
| Fixed objects/run-off the | slippery surface | 1-overlay pavement (friction course) |
| road crashes |  | 2-chip and seal or slurry seal roadway |
|  |  | 3-groove pavement surface |
|  |  | 4-provide adequate drainage and/or improve |
|  |  | crown |
|  |  | 5-set appropriate speed limit on approaches ** |
|  |  | 6-use "SLIPPERY WHEN WET" sign * |
|  | roadway design is no longer | 1-widen lanes and/or shoulders |
|  | adequate for traffic conditions | 2-relocate or remove islands |
|  |  | 3-flatten side slope |
|  |  | 4-provide proper superelevation on curve |
|  |  | 5-construct more gradual horizontal curve |
|  |  | 6-install post-mounted delineators on horizontal |
|  |  | curve |
|  |  | 7-install chevron alignment sign on horizontal curve |
|  | poor delineation | 1-install/improve warning signs * |
|  |  | 2-install/improve pavement markings |
|  |  | 3-install roadside delineators or chevron alignment |
|  |  | signs * |
|  | driver has inadequate warning | 1-install curve warning sign * |
|  | of roadway alignment change | 2-install advisory speed plate or curve warning |
|  |  | sign(s) * |
|  |  | 3-install large arrow warning sign * |
|  | excessive speed | set appropriate speed limit ** |
|  | inadequate roadway lighting | install/improve roadway lighting |
|  | poor visibility of traffic control | increase sign size |
|  | devices |  |

Table b-1 (CONT.):  general countermeasures for crash patterns and their probable causes

| CRASH PATTERN | PROBABLE CAUSE | COUNTERMEASURE |
| --- | --- | --- |
| Fixed objects/run-off the | inadequate shoulder | upgrade roadway shoulder |
| road crashes (cont'd) |  |  |
|  | inadequate channelization | provide adequate channelization |
|  | inadequate pavement | repair road surface |
|  | maintenance |  |
| Collisions with parked | high rate of parking turnover | 1-change from angle to parallel parking |
| vehicles or vehicles |  | 2-provide short-term off-street parking |
| being parked |  | 3-prohibit parking |
|  |  | 4-restrict parking during rush hours |
|  |  | 5-reroute through traffic |
|  |  | 6-create one-way streets |
|  | roadway design is not | 1-widen lanes |
|  | adequate for traffic conditions | 2-change from angle to parallel parking |
|  |  | 3-prohibit parking |
|  |  | 4-restrict parking during rush hours |
|  |  | 5-reroute through traffic |
|  |  | 6-set appropriate speed limit on traveled way ** |
|  | inadequate parking | restrict parking near corner/ |
|  | clearance at driveway | crosswalk/driveway |
|  | excessive speed | set appropriate speed limit ** |
|  | inadequate or improper | mark parking stall limits |
|  | pavement markings |  |
|  | angle parking | convert angle to parallel parking |
|  | illegal parking | 1-increase enforcement |
|  |  | 2-prohibit parking |
|  |  | 3-create off-street parking |

Table b-1 (CONT.):  general countermeasures for crash patterns and their probable causes

| CRASH PATTERN | PROBABLE CAUSE | COUNTERMEASURE |
| --- | --- | --- |
| Collisions at driveways | improperly located driveway | 1-regulate minimum spacing of driveways |
|  |  | 2-regulate minimum corner clearances |
|  |  | 3-move driveway to side street |
|  |  | 4-install curb to define driveway location |
|  |  | 5-combine adjacent driveways |
|  | inadequate sight distance | 1-remove sight obstructions |
|  |  | 2-restrict parking near driveway |
|  |  | 3-install/improve lighting at driveways |
|  |  | 4-set appropriate speed limit ** |
|  |  | 5-improve vertical curve |
|  | left-turn vehicles | 1-install median barrier |
|  |  | 2-install continuous two-way left-turn lane |
|  |  | 3-install protected left-turn bays |
|  | right-turn vehicles | 1-install right-turn lanes |
|  |  | 2-restrict parking near driveways |
|  |  | 3-increase roadway width |
|  |  | 4-widen through-lanes |
|  |  | 5-increase driveway curb radii |
|  | excessive speed | set appropriate speed limit ** |
|  | large volume of through traffic | 1-move driveway to side street |
|  |  | 2-construct a local service road |
|  |  | 3-re-route through traffic |
|  | large volume of driveway | 1-install signal at driveway |
|  | traffic | 2-provide acceleration and/or deceleration lanes |
|  |  | 3-widen and/or channelize driveway |
|  |  | 4-construct additional driveway |
|  |  | 5-change to one-way driveway |
|  | inadequate roadway lighting | improve roadway lighting |

Table b-1 (CONT.):  general countermeasures for crash patterns and their probable causes

| CRASH PATTERN | PROBABLE CAUSE | COUNTERMEASURE |
| --- | --- | --- |
| Crashes on wet | water ponding on roadway | 1-provide adequate drainage and/or improve crown |
| pavement |  | 2-remove turf or other drainage impediments from |
|  |  | shoulder |
|  | slippery surface | 1-overlay pavement (friction course ) |
|  |  | 2-chip and seal or slurry seal roadway |
|  |  | 3-groove pavement surface |
|  |  | 4-set appropriate speed limit ** |
|  |  | 5-use "SLIPPERY WHEN WET" sign * |
|  |  | 6-provide adequate drainage |
|  |  | 7-improve skid resistance |
|  | inadequate/improper | install raised/reflectorized pavement markers |
|  | pavement markings |  |
| Crashes at night | poor visibility | 1-install/improve street lighting |
|  |  | 2-install/improve reflectorized signs |
|  |  | 3-install/improve reflectorized pavement markings |
|  |  | 4-remove distracting commercial lighting |
|  |  | or other sources of glare |
|  | poor visibility of traffic control | 1-install/improve warning signs * |
|  | devices | 2-improve roadway lighting |
|  |  | 3-install/improve delineation |
|  | inadequate signing | 1-upgrade traffic control devices * |
|  |  | 2-provide illuminated signs |
|  |  | 3-install chevron alignment sign on horizontal curve |
|  | inadequate delineation | 1-install/improve warning signs * |
|  |  | 2-provide raised markings |
|  |  | 3-install/improve delineation |
|  |  | 4-install post-mounted delineators on horizontal |
|  |  | curve |

Table b-1 (CONT.):  general countermeasures for crash patterns and their probable causes

| CRASH PATTERN | PROBABLE CAUSE | COUNTERMEASURE |
| --- | --- | --- |
| Crashes at night | inadequate channelization | 1-install/improve warning signs * |
| (cont'd) |  | 2-provide raised markings |
|  |  | 3-install/improve delineation |
|  |  | 4-install/improve pavement markings |
| Collisions at railroad | inadequate sight distance | 1-remove sight obstructions |
| grade crossing |  | 2-improve/install advance warning signs * |
|  |  | 3-provide stop sign * |
|  |  | 4-improve/install pavement markings * |
|  |  | 5-reduce grade |
|  |  | 6-install train actuated signals * |
|  |  | 7-install overhead flashing beacon * |
|  |  | 8-install automatic crossing gates |
|  |  | 9-improve intersection approach angle |
|  |  | 10-install bridge or tunnel |
|  | poor visibility | 1-install/improve lighting at crossing |
|  |  | 2-install larger, reflectorized signs |
|  | slippery surface | 1-improve drainage |
|  |  | 2-install skid-resistant surface |
|  | excessive vehicle or train | 1-set appropriate speed limit ** |
|  | speed | 2-reduce train speed near crossing |
|  | inadequate/improper | 1-add markings to supplement signs |
|  | pavement markings | 2-install limit lines |
|  |  | 3-install/improve pavement markings |
|  | improper traffic signal | re-time signal |
|  | preemption timing |  |
|  | improper signal or gate | re-time automatic flashers or flashers with gates |
|  | warning time |  |

Table b-1 (CONT.):  general countermeasures for crash patterns and their probable causes

| CRASH PATTERN | PROBABLE CAUSE | COUNTERMEASURE |
| --- | --- | --- |
| Collisions at railroad | rough crossing surface | improve crossing surface |
| grade crossing (cont'd) |  |  |
|  | sharp crossing angle | rebuild crossing with proper angle |
| Sideswipe or head-on | roadway design is no longer | 1-install/improve center line markings * |
| collisions between | adequate for traffic conditions | 2-channelize intersection |
| vehicles traveling in |  | 3-widen lanes and/or shoulders |
| opposite directions |  | 4-remove constriction such as parked vehicles |
|  |  | 5-install median barrier |
|  |  | 6-create one-way streets |
|  | excessive speed | set appropriate speed limit ** |
|  | inadequate/ improper | install/improve pavement markings |
|  | pavement markings |  |
|  | inadequate shoulder | upgrade roadway shoulder |
|  | inadequate channelization | 1-provide adequate channelization |
|  |  | 2-provide turn lane |
|  |  | 3-install acceleration/deceleration lane |
|  |  | 4-install median barrier |
|  | inadequate signing | 1- install illuminated street name signs |
|  |  | 2-install advance guide sign * |
|  | inadequate pavement | repair road surface |
|  | maintenance |  |
| Lane change, sideswipe | inadequate traffic control | 1-install/improve pavement lane lines |
| or turning collisions | devices | 2-install advance route identification signs or street |
| between vehicles |  | name signs |
| traveling in the same |  |  |
| direction |  |  |

Table b-1 (CONT.):  general countermeasures for crash patterns and their probable causes

| CRASH PATTERN | PROBABLE CAUSE | COUNTERMEASURE |
| --- | --- | --- |
| Lane change, sideswipe | roadway design is no longer | 1-widen lanes and/or shoulders |
| or turning collisions | adequate for traffic conditions | 2-remove constriction such as parked vehicles |
| between vehicles |  | 3-channelize intersection |
| traveling in the same |  | 4-provide turning bay for high-volume driveway |
| direction (cont'd) |  | 5-install continuous two-way left turn lane |
|  |  | 6-set appropriate speed limit ** |
|  | excessive speed | set appropriate speed limit ** |
|  | inadequate/improper | install/improve pavement markings |
|  | pavement markings |  |
|  | inadequate shoulder | upgrade roadway shoulder |
|  | inadequate channelization | 1-provide adequate channelization |
|  |  | 2-provide turn lane |
|  |  | 3-install acceleration/deceleration lane |
|  | inadequate pavement | repair road surface |
|  | maintenance |  |
|  | inadequate signing | 1-install illuminated street name signs |
|  |  | 2-install advance guide sign * |
| Left turn collisions at | restricted sight distance | 1-provide left-turn signal phase |
| intersections |  | 2-provide adequate channelization |
|  |  | 3-remove sight obstructions |
|  |  | 4-provide turn lane |
|  |  | 5-install/improve warning sign * |
|  |  | 6-set appropriate speed limit ** |
|  | absence of left-turn phase | add left-turn signal phase |

Table b-1 (CONT.):  general countermeasures for crash patterns and their probable causes

| CRASH PATTERN | PROBABLE CAUSE | COUNTERMEASURE |
| --- | --- | --- |
| Left turn collisions at | large volume of left-turn | 1-create one-way street |
| intersections (cont'd) | traffic | 2-install left-turn lane |
|  |  | 3-add left-turn signal phase |
|  |  | 4-prohibit left-turn |
|  |  | 5-re-route left-turn traffic |
|  |  | 6-provide adequate channelization |
|  |  | 7-install stop sign * |
|  |  | 8-adjust signal phase sequence |
|  |  | 9-provide turning guidelines for multiple left-turn |
|  |  | lanes |
|  |  | 10-install new traffic signal * |
|  |  | 11-re-time signal |
|  | yellow phase too short | 1-adjust yellow change interval |
|  |  | 2-add all-red interval |
|  | excessive speed | set appropriate speed limit ** |
| Right-turn collisions at | inadequate turning path | increase curb radii |
| intersections |  |  |
|  | restricted sight distance | 1-remove sight obstructions |
|  |  | 2-add "NO TURN ON RED" signs if signalized * |
|  |  | 3-set appropriate speed limit on approaches ** |
| Pedestrian crashes at | sidewalk too close to the | move sidewalk laterally away from street |
| driveways | roadway |  |
| * Refer to Manual on Uniform Traffic Control Devices for proper application and warrants. |  |  |
| ** Spot speed study should be conducted to justify speed limit. |  |  |

Table b-1 (CONT.):  general countermeasures for crash patterns and their probable causes

APPENDIX C

COLLECTION OF TRAFFIC DATA

This appendix explains how to conduct several types of studies used to collect traffic data.

INTERSECTION VOLUME STUDIES AND ENTERING ADT ESTIMATES

One of the most important reasons for conducting an intersection traffic count is to collect the information needed to estimate the entering Average Daily Traffic (ADT).  To conduct an intersection traffic count, record the vehicle paths of entry and departure at the location.  Occasionally, it is necessary to classify vehicles by type and to count pedestrians and cyclists.

Due to staff and cost constraints, the manual counting period duration is limited, and the counts are samples of actual traffic volumes.  The sampling period for manual counting may range from 1 to 12 hours.  Mechanical or automated equipment can provide longer sample periods, from a few hours to a full year.

Manual Traffic Counts

The following sections explain the recommended procedures for obtaining accurate manual traffic counts.

What to Count

Use the following guidelines for counting and classifying:

- Unless otherwise directed, only count vehicles entering the intersection.  When required, tally pedestrians and cyclists.

- Record each vehicle according to the direction from which it approaches the intersection and whether it turns right or left, or goes straight.  Count pedestrians each time a crosswalk is used.

- Count U-turns as left turns.

- Classify vehicles as:

- Passenger vehicles:  cars, vans, smaller trucks (e.g. pick-ups), and motorcycles,

- Trucks:  larger trucks (six or more tires) and semi-trailer or combination trucks, and

- Buses:  commercial and school buses.

Guidelines for detailed vehicle classification studies are available from the MoDOT Technology Transfer Assistance Program (TTAP) office.  The TTAP office can also provide information on the community traffic counting program and community traffic maps.

Tally Sheet

Manually record vehicle volume and turning movement counts at intersections using the Traffic Count Field Sheet shown in Figure C-1.  The tally sheets have 12 rectangles for recording vehicle movements and 4 squares for recording pedestrian crossing activity.  Before beginning the counts, enter the street name, date, time, and other related information on the tally sheet.  It is best to prepare all sheets that will be needed prior to the first counting period.  A single field sheet could be used for whatever time period is desired; however it is recommended that a new sheet be started every 15 minutes during the study.

To record pedestrians and vehicles, use a tally system consisting of four vertical marks with every fifth mark placed diagonally across the four marks (i.e.        ).  Symbols such as a “T” for a truck, “B” for a bus, and “SB” for a school bus should be utilized to classify vehicles.

Note any unusual events that affect the traffic flow during the counting period, and their duration.  If an incident occurs that substantially disrupts traffic flow (in a way that would eliminate the usefulness of the study), stop the count and conduct the study at another time.

Suggested Equipment

The observer(s) conducting the traffic count should have the following equipment on site:

- A watch with a second hand or a digital watch,

- Several pencils with erasers,

- A pencil sharpener,

- A clip board, and

- An accumulating register (optional).

Procedures

Most intersection counts require two observers.  However, one observer can usually count a low-volume intersection.  When two people are counting traffic at a four-leg intersection, they should be positioned in diagonally opposite quadrants (e.g. the northwest and southwest corners).  Each observer should tally vehicles entering on two approaches.  The observer must be inconspicuous, so his or her presence does not affect traffic operations.

Figure C-1:  INTERSECTION TRAFFIC COUNT FIELD SHEET

Hand-operated accumulating registers can be used to ease the tallying process.  These registers are available in configurations representing intersection turning movements.  Running totals are recorded at appropriate sampling intervals.

When scheduling traffic counting periods, care should be taken to avoid unusually busy or idle times.  Data should not be gathered on a weekend, a Friday, the day of a special event, or a holiday.

Count Summaries

Traffic counts from the field study should be summarized as illustrated in Figure C-2, the Turning Movement Count Summary.  The traffic counts in this figure are for the HAL Manual example location, the intersection of Lincoln and Third Street.

In this example, the counts were taken during a Tuesday evening peak hour.  To arrive at the estimated intersection entering ADT, an adjustment factor of 10 was applied to the one-hour counts on each incoming approach.  Then, as shown at the bottom of Figure C-2, the ADT estimates from each incoming approach were summed to form the “Intersection ADT Estimate.”

Automated Traffic Counts

Automated traffic counts enable an agency to gather large amounts of volume data at a reasonable cost.  For a long study, automated traffic counts are less expensive than manual counts because labor costs are lower.  The main drawback to the automated system is the possibility that the equipment could fail due to malfunctions or vandalism.

Equipment

The many different types of automated counters can be divided into three categories:

- Portable Counters – Portable counters are usually used for short periods of time (24 hours).  The most common sensors in these counters include pneumatic road tubes, piezoelectric strips, tape switches, or temporary induction loop detectors.  Count readers range from simple accumulating counters to micro-computer-driven classification counters.

- Permanent Counters – Permanent counters are used for long-term projects that can last for a year or more.  These counters use the same type of recording components as the portable counters, but the sensors are more permanent.  The most common type of sensor is an induction loop, which is installed in the pavement.

- Videotapes – Videotapes give the observer an exact account of the number of vehicles during the study.  They also provide the observer with information that can

Figure C-2:  VEHICLE TURNING COUNT SUMMARY AND ADT ESTIMATE

- be reused in other studies.  After the tapes are recorded, someone must watch the tape and manually count the vehicles.  Agencies typically use videotapes only if very high accuracy is needed.  An alternative to manual counting from the tapes is video imaging, which counts automatically.  The cost of video systems is falling, and the system is 80 to 95% accurate during the day or night, making it a more feasible option in the future.

Selecting the Count Location

Use the following guidelines to ensure the location selected for the traffic count is appropriate:

- Deploy sensors at right angles to traffic flow.

- For directional counts, place sensors at least 1 foot away from the centerline of the roadway.

- Fasten the sensor securely to the pavement with nails, clamps, tape, and/or adhesives made especially for this purpose.

- At intersections or near driveways, place sensors where double counting of turning vehicles can be avoided.

- Locate the count reader near a signpost or tree and secure it with a lock and chain, or place it in a locked signal cabinet to prevent vandalism.

- Keep the cable or tube that connects the sensor to the recorder as short as possible.

- Record sensor placement by noting the physical location on a condition diagram sketch.

- Use a test vehicle to ensure that bi-directional counts are recording the proper direction.

- Set the count interval to ensure that totals will occur on the hour or day to make the data more compatible with other counts.

- Note the time that counter operation begins.

- Check the installation periodically to ensure that it is in place and functioning properly.  In cold weather, check sensors whenever it snows to ensure that snowplows do not remove the sensors from the roadway.

- Do not place sensors across parking lanes, where a parked vehicle could activate the sensor continuously.  Parking lanes may not always be marked.

- Avoid placing sensors on pavement expansion joints, sharp pavement edges, or curves.

Once the counts are complete, use the form in Figure C-2 to summarize the data.

Conducting Spot Speed Studies and Setting Speed Limits

A spot speed study measures the individual speeds of a sample of vehicles passing a specific point on a roadway.  The individual vehicle speeds are used to estimate the speed distribution of the entire traffic stream at that location.  Speeds are determined using an observer with a stopwatch, radar, or automated traffic detectors.

Spot speed studies are used to help determine the appropriate speed limit and to evaluate sight distance problems at intersections and other critical locations.

Selection of Study Location and Time

To conduct a spot speed study, choose a mid-block location away from the influence of stop signs, signals, major driveways, and sharp curves.  The site must have an observation point near the roadway, where a vehicle with radar equipment can be concealed or made inconspicuous to approaching drivers.

Perform spot speed studies in good weather and under normal traffic conditions.  Usually, speed studies are conducted during off-peak hours.  A recommended method is to sample for one or two hours, three times during a day.  Under most circumstances, the three studies should be conducted from 9:00 AM to 11:30 AM; 1:30 PM to 4:00 PM; and 7:00 PM to 10:00PM.

Study Procedure

Measure at least 100 vehicle speeds, preferably more, during a spot speed study.  Low-volume roads might require more than one day of observation to obtain the required minimum sample size.

Select vehicles to be measured at random, or according to a predetermined pattern, so the data are not biased.  Determine the vehicle selection pattern before beginning the field study.  For instance, the observer could measure every fourth or fifth vehicle whenever possible.

If vehicle selection is not random or according to a pre-determined pattern, then record only the speeds of free-flowing vehicles.  Free-flowing vehicles are those whose speeds are not influenced by preceding vehicles.  Select trucks for speed observation in proportion to their presence in traffic.  Observers should avoid the temptation to measure only the fastest vehicles. Observations are usually recorded by tallying the number of vehicle speeds that occur within a certain speed interval, such as a two- or five-mph interval.

Data Analysis

Traffic speed data may be summarized for analysis purposes, as shown in Table C-1.  The example speed data in this table contain 120 observations.  The observations are grouped into two-mph intervals, and the intervals range from 20 mph to 41.9 mph.

|  | A |  | B |  | C |  | D |  | E |  | F |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Speed |  |  |  | Cumulative |  |  |  | Cumulative |  |  |  |
|  | Interval |  | Number |  | Number |  | Percent |  | Percent of |  | 10-mph |  |
|  | in mph |  | Observed |  | Observed |  | Observed |  | Observations |  | Pace |  |
|  | 20 to 21.9 |  | 3 |  | 3 |  | 2.5 |  | 2.5 |  |  |  |
|  | 22 to 23.9 |  | 3 |  | 6 |  | 2.5 |  | 5 |  |  |  |
|  | 24 to 25.9 |  | 6 |  | 12 |  | 5 |  | 10 |  |  |  |
|  | 26 to 27.9 |  | 12 |  | 24 |  | 10 |  | 20 |  | 12 |  |
|  | 28 to 29.9 |  | 18 |  | 42 |  | 15 |  | 35 |  | 18 |  |
|  | 30 to 31.9 |  | 27 |  | 69 |  | 22.5 |  | 57.5 |  | 27 |  |
|  | 32 to 33.9 |  | 24 |  | 93 |  | 20 |  | 77.5 |  | 24 |  |
|  | 34 to 35.9* |  | 13 |  | 106 |  | 10.8 |  | 88.3* |  | 13 |  |
|  | 36 to 37.9 |  | 8 |  | 114 |  | 6.7 |  | 95 |  |  |  |
|  | 38 to 39.9 |  | 4 |  | 118 |  | 3.3 |  | 98.3 |  |  |  |
|  | 40 to 41.9 |  | 2 |  | 120 |  | 1.7 |  | 100 |  |  |  |
|  | * The 85th percentile is in the interval from 34 to 35.9. |  |  |  |  |  |  |  |  |  |  |  |

Table C-1:  SPOT SPEED STUDY DATA ANALYSIS

The number of vehicle speeds observed in each interval is recorded in the column B.  The cumulative number observed (column C) is calculated by adding the number observed in each speed interval to the previous number observed.  The percent observed in each two-mph speed interval (column D) is calculated by dividing each number in column B by last number in column C.  The cumulative percent of observations (column E) is calculated by adding the percent observed in each speed interval to the previous percent observed.  The percent corresponding to the last speed interval should be 100%.  Column C is also used for finding the 85th percentile speed, discussed in a following paragraph.

Two of the most frequently used traffic speed characteristics to be computed from spot speed studies are the “85th Percentile Speed” and the “10-mph pace.”

85th Percentile Speed

The 85th percentile speed is the speed below which 85% of the observed vehicles travel.  It is the most important factor in speed zoning practice for communities.  Traffic engineers generally assume that the majority of drivers will be reasonable and will travel at a speed that is safe and proper for the exisiting conditions.  However, this practice does recognize that a few drivers will be operating at a speed somewhat greater than the speed considered appropriate by a large number of drivers.

For the data in Table C-1, the 85th percentile is contained within the interval from 34.0 to 35.9 mph.  This can be verified by noting that 77.5% of the observations were accumulated when the speed reached 33.9 mph, and that 88.3% were accumulated after 35.9 mph.  This provides a good indication that a 35-mph speed limit would be appropriate.

10-mph Pace

The 10-mph pace is the 10-mph range of speeds that includes the greatest number of observations.  The top limit of the 10-mph pace indicates the highest speed many drivers prefer, and it may be used to confirm the value selected according to the 85th percentile analysis.

Column F in Table C-1 identifies the 10-mph pace.  For this speed study, the 10-mph pace is between 26.0 and 35.9 mph, since that 10-mph range contains the largest number of vehicles (12+18+27+24+13 = 94 vehicles).

Since the upper limit of the 10-mph pace is 35.9 mph, then the choice of 35 mph for the speed limit is supported.

Several other factors to consider when setting speed limits include:

- Crash experience,

- Presence of restricted sight distances,

- Design speed,

- Roadway surface characteristics,

- Extent of turning movements,

- Parking conditions, and

- Number of pedestrians.

It is important not to establish a speed limit that is too high or too low.  Speed limits that appear highly unreasonable to motorists may lead to driver frustration and disregard for all traffic control devices.  Speed limits must be posted in increments of 5 mph, using speeds such as 30, 35, 40 mph, and not at unusual limits like 33 mph.

INTERSeCTION SIGHT DISTANCE STUDIES

Sight distance studies at intersections help to identify hazardous locations.

Sight Distance for Intersections With Yield or No Control

Sight distance studies for intersections with yield or no control are essentially triangle analyses.  A driver approaching an intersection where direction priority is not assigned (no control) should have an unobstructed view of the entire intersection and sufficient length along the crossroad to avoid a collision.  Therefore, an unobstructed line of sight must be provided to allow a driver to detect a vehicle approaching on a conflicting path.

The required sight distances for safe operation when approaching an intersection are shown in Figure C-3.  The distances represented as “a” and “b” in this figure should provide sufficient time for drivers to adjust their speeds and, if necessary, stop their vehicles prior to entering the intersection.

FIGURE C-3:  INTERSECTION SIGHT TRIANGLE FOR SAFE APPROACH SPEED

Table C-2 lists the safe stopping distances for vehicles approaching the intersection at different speeds.  For example, if the speed of Vehicle A was 20 mph, and the speed of Vehicle B was 45 mph, then the line of sight drawn in Figure C-3 must be unrestricted when Vehicle A is 125 feet from the intersection and Vehicle B is 400 feet from the intersection.

| Posted Speed, | Stopping Sight |
| --- | --- |
| 85th Percentile Speed, | Distance, |
| or Design Speed, in mph | in feet |
| 20 | 125 |
| 25 | 150 |
| 30 | 200 |
| 35 | 250 |
| 40 | 325 |
| 45 | 400 |
| 50 | 475 |
| 55 | 550 |
| 60 | 650 |
| 65 | 725 |
| 70 | 850 |

Table C-2:  RECOMMENDED STOPPING SIGHT DISTANCE                               (adapted from AASHTO 1990)

The recommended procedures for determining safe approach speeds at intersections with no control are:

- Determine the minimum required stopping sight distance from Table C-2 for all intersection roadways, using the largest of the 85th percentile speed, the speed limit, or the design speed on the approach.

- Provide an observer with a sighting rod that is 3.5 feet high (representing driver eye height) and an assistant with a target rod 4.25 feet high (representing the top of a car).  The observer and assistant should position themselves on different approaches at the appropriate stopping distance from the intersection.

- Hold both rods vertically on the road at their respective stopping distances.  The observer looking over the top edge of the sighting rod should determine whether the top of the target rod is visible.  If the target rod is visible, the visibility triangle is satisfactory for the pair of approaches.

- If the top of the target rod is not visible, then the assistant with the target rod should walk toward the intersection until the top of the rod becomes visible to the observer.  This position should be marked and the distance to the intersection measured.  The safe speed for the approach can be determined by referring to the stopping distances listed in Table C-2.

- Repeat the intersection sight triangle study for all approach legs, considering traffic approaching from both the right and left.

- Conduct sight distance measurements during, or at least with consideration given to, possible short-term adverse conditions.  For example, trees, shrubs, and parked cars can all affect sight lines.

- If the available stopping sight distance is not equal to or greater than that required for safe vehicle operation, the obstruction within the triangle should be removed or lowered.  If this is not possible, other options include reducing the speed on one or both of the roadways to be compatible with the safe approach speed, or installing a STOP sign.

Sight Distances on Controlled Approaches

Instructions for locating intersection traffic control devices such as STOP signs or YIELD signs are provided in the Manual on Uniform Traffic Control Devices (MUTCD).  If the visibility of a STOP sign or YIELD sign at a location is restricted, a warning sign must be installed in advance of the regulatory sign.

STOP signs and YIELD signs should be visible to approaching drivers for the safe stopping sight distances in Table C-2.  These distances may be checked in the field using a sighting rod 3.5 feet high.  The sighting rod should be placed at the appropriate safe stopping distance on the approach as required by the approach speed.  If the intersection sign is not visible from the sighting rod, a warning sign must be installed.

Since warning sings are primarily for the benefit of the driver who is unacquainted with the road, care must be given to the placement of such signs.  Table C-3 contains minimum advance sign placement distances for conditions where a driver will likely be required to stop.

|  |  | Posted Speed or | Warning Sign Location |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 85th Percentile | in Advance of |  |  |
|  |  | Speed, in mph | Regulatory Sign, in feet |  |  |
|  |  | 20 | 100* |  |  |
|  |  | 25 | 100* |  |  |
|  |  | 30 | 100 |  |  |
|  |  | 35 | 150 |  |  |
|  |  | 40 | 225 |  |  |
|  |  | 45 | 300 |  |  |
|  |  | 50 | 375 |  |  |
|  |  | 55 | 450 |  |  |
|  |  | * At low speeds, sign location may depend on |  |  |  |
|  |  | physical conditions at the site or view obstruction. |  |  |  |

Table C-3:  GUIDE FOR ADVANCE WARNING SIGN PLACEMENT           (adapted from AASHTO 1990)

Leaving Two-Way Stop Intersections

Safe sight distances must be provided for a driver to turn onto or cross a highway from each STOP controlled approach where major road traffic does not stop.  Sight distances to the left and right must allow a stopped car to perform an entry or crossing maneuver without risking a collision with a vehicle that may appear just after the driver decides to proceed.

Assume that a car waiting at a STOP sign will be positioned so the vehicle front bumper is 10 feet from the near edge of the pavement on the crossroad.  To determine if the line of sight from a stopped car is adequate, measure sight distances from a driver’s eye height (3.5 feet above the pavement) to the top of the object representing an on-coming car (4.25 feet above the pavement).  Table C-4 lists the sight distances required for a passenger car to turn safely onto or cross a two-lane highway.

If a safe distance does not exist along an approach, then take corrective measures to improve the sight distance, provide warnings to approaching drivers, or reduce speeds on the major roadway.

|  |  |  | Sight Distance (in Feet) Along Major Road for Maneuver Indicated |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Speed |  | Cross | Right Turn to Enter | Left Turn to Enter Roadway in |  |  |  |
|  | on |  | the | Roadway in Front of | Front of Vehicle Approaching |  |  |  |
|  | Major |  | Major | Vehicle Approaching |  |  |  |  |
|  | Road |  | Road | From the Left* |  |  |  |  |
|  | (mph) |  |  |  | From the Left* |  | From the Right* |  |
|  | 25 |  | 240 | 295 | 260 |  | 295 |  |
|  | 30 |  | 285 | 375 | 310 |  | 375 |  |
|  | 35 |  | 335 | 470 | 360 |  | 470 |  |
|  | 40 |  | 385 | 575 | 410 |  | 575 |  |
|  | 45 |  | 430 | 710 | 460 |  | 710 |  |
|  | 50 |  | 480 | 845 | 510 |  | 845 |  |
|  | 55 |  | 525 | 990 | 560 |  | 990 |  |
|  | * Distances shown for turning maneuvers assume an approaching vehicle |  |  |  |  |  |  |  |
|  | will reduce its speed from the design speed to 85% of design speed. |  |  |  |  |  |  |  |

TABLE C-4:  SIGHT DISTANCES REQUIRED FOR A PASSENGER CAR STOPPED AT AN INTERSECTION TO CROSS OR TURN ONTO A MAJOR ROAD (adapted from AAShto 1990)

TRAFFIC CONFLICT STUDIES

A traffic conflict is an event involving two or more road users.  A conflict occurs when the action of one user, such as a change in direction or speed, causes the other to make a sudden, evasive maneuver, such as swerving or braking, to avoid a collision.

A secondary traffic conflict occurs when the second vehicle makes an evasive maneuver, placing another road user (third vehicle) in danger of a collision.  Generally, the road users are motorists, but pedestrians and cyclists may also be affected.

There are several categories of intersection traffic conflicts, and they are classified according to the vehicle maneuvers involved.  In each traffic conflict category, the road users must have been on a collision course.

If traffic conflicts are not addressed in a timely manner, the result is frequently a crash.  A “near-miss” situation occurring without braking or evasive maneuvers is also considered a traffic conflict.

Traffic Conflict Types

A general knowledge of traffic conflict types is necessary before an observer conducts an on-site conflict study.  Figures C-4 through C-16 show examples of the types of traffic conflicts most likely to be observed.  Note that the conflicts are named from the perspective of the observer, represented by an “X” in the figures.

- An opposing left-turn conflict occurs when an on-coming vehicle makes a left-turn, placing another vehicle going in the opposite direction in danger of a head-on or broadside collision (Figure C-4).

figure c-4:  OPPOSING LEFT-TURN CONFLICT

- A conflict occurs when a vehicle on the left-hand cross street makes a left-turn, placing a second vehicle on the main street in danger of a broadside or rear-end collision (Figure C-5).

figure c-5:  LEFT-TURN CROSS-TRAFFIC FROM LEFT CONFLICT

- A conflict occurs when a vehicle on the left-hand cross street crosses in front of a second vehicle on the main street, placing it in danger of a broadside collision (Figure C-6).

Figure c-6:  THROUGH-traffic CROSS-TRAFFIC FROM LEFT CONFLICT

- A conflict occurs when a vehicle on the left-hand cross street turns right across the center of the main street roadway into an opposing lane, placing the vehicle in that lane in danger of collision (Figure C-7).  Note that the first driver must cross the centerline for a conflict to exist.

figure c-7:  RIGHT-TURN CROSS-TRAFFIC FROM LEFT CONFLICT

- A conflict between a vehicle turning left and traffic in the same direction occurs when the first vehicle slows to make a left-turn, thus placing a second, following vehicle in danger of a rear-end collision (Figure C-8).

figure c-8:  LEFT-TURN SAME DIRECTION CONFLICT

- A conflict between a slow vehicle and traffic in the same direction occurs when the first vehicle slows while approaching or passing through an intersection, placing a second, following vehicle in danger of a rear-end collision (Figure C-9).

figure c-9:  SLOW-VEHICLE SAME DIRECTION CONFLICT

- A conflict between vehicles in the same lane occurs when the first vehicle changes from one lane to another, thus placing a second, following vehicle in the new lane in danger of a rear-end collision (Figure C-10).

figure c-10:  LANE-CHANGE SAME-DIRECTION CONFLICT

- A conflict between traffic turning right and traffic in the same direction occurs when the first vehicle slows to make a right turn, thus placing the second, following vehicle in danger of a rear-end collision (Figure C-11).

figure c-11:  RIGHT-TURN SAME DIRECTION CONFLICT

- A conflict occurs when a vehicle on the right-hand cross street makes a left-turn, placing a second vehicle in danger of having a broadside collision with the turning vehicle (Figure C-12).

figure c-12:  LEFT-TURN CROSS-TRAFFIC FROM RIGHT CONFLICT

- A conflict occurs when a left-turning vehicle on the right-hand cross street crosses in front of a second vehicle on the main street, placing it in danger of a broadside collision (Figure C-13).

figure c-13:  THROUGH CROSS-TRAFFIC FROM RIGHT CONFLICT

- A conflict occurs when a vehicle on the right-hand cross street makes a right-turn, thus placing a second vehicle, on the main street, in danger of making a broadside or rear-end collision (Figure C-14).

figure C-14:  RIGHT-TURN CROSS-TRAFFIC FROM RIGHT CONFLICT

- An example of a secondary conflict is a situation similar to RIGHT-TURN CROSS-TRAFFIC FROM RIGHT, except a third vehicle is involved.  The third vehicle is in danger of colliding with the rear-end of the vehicle it is following (Figure C-15).

figure c-15:  SECONDARY TRAFFIC CONFLICT EXAMPLE – RIGHT-TURN CROSS-TRAFFIC FROM RIGHT

- A pedestrian conflict occurs when a pedestrian crosses in front of a vehicle, creating a potential collision.  The pedestrian could be in the near-side or far-side crosswalk.  Pedestrian movements involving right-turn and left-turn vehicles are not considered conflicts if the pedestrians have the right-of-way, as in a “WALK” phase (Figure C-16).

figure c-16:  PEDESTRIAN CONFLICT

The Traffic Conflict Summary Sheet

The traffic conflict summary sheet in Figure C-17 may be used for recording and summarizing conflict counts.  Each conflict classification has two columns for recording observations.  Record conflicts with pedestrians, cyclists, or vehicles from access points near the intersection in the last column.

Fill out all heading information prior to beginning the conflict study.  The diagram in the upper right corner displays the approach leg number.  For example, traffic approaching the site from the North is on leg 1; traffic from the East is on leg 3; etc.  Use a separate form for each leg observed at the intersection.

Coordinating the Traffic Conflict Study

A traffic conflict study includes counting conflicts and collecting other data needed to make a complete study of the location.  These auxiliary data may include intersection condition diagrams, on-site observation reports, traffic volume counts, and sight distance studies.  Conflict studies should be performed during dry conditions, unless the study is specifically designed for wet conditions.

Traffic Conflict Study Team

The number of observers needed to conduct a conflict survey depends on the number of conflicts and amount of data needed.  Usually, the team consists of two observers in a vehicle – one to collect conflict data and one to collect traffic volume data.

Observer Locations

Upon arriving at the site, the study team members should familiarize themselves with the location, noting the traffic movements to be observed.  At three- and four-leg signalized locations, observations are usually taken on all approaches.  At an unsignalized intersection, observations are made only on approaches where vehicles have the right-of-way.

Since braking and weaving actions identify conflicts, it is necessary to place the observer sufficiently far back on the approach to observe these maneuvers.  A distance of 100 to 300 feet back from the intersection facing the direction of traffic movement is suggested.

If either observer is to sit in a vehicle, it should be parked off the road wherever possible.  If on-street parking is permitted, check for an adequate spot to conduct the study that will not disturb traffic movements or interfere with any sight distances.  If parking is not available, the observers will have to conduct the study outside of the vehicle, being as inconspicuous as possible.  In all instances, the observer must not use a vehicle that could be recognized as a police or other official car.

| Location _____________________________________________ Date_______________ 8 1=N 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Observer ____________________________________________ Day _______________ Leg Number: _______ 7 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| [C = Conflict SC = Secondary Conflict] Time of Study: From: _________ To: ____________ 6 5 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Time Start  End | OPPOSING |  | FROM LEFT |  |  |  |  |  | SAME DIRECTION |  |  |  |  |  |  |  | FROM RIGHT |  |  |  |  |  | OTHER |  |
|  | Left Turn |  | Left Turn |  | Thru |  | Right Turn |  | Left Turn |  | Slow Vehicle |  | Lane Change |  | Right Turn |  | Left Turn |  | Thru |  | Right Turn |  |  |  |
|  | C | SC | C | SC | C | SC | C | SC | C | SC | C | SC | C | SC | C | SC | C | SC | C | SC | C | SC | C | SC |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUM C+SC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COMMENTS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

figure c-17:  intersection traffic conflict summary (adapted from parker and zeeger 1989)

Once the observation positions are determined, all forms should be prepared and double-checked before data collection begins.  If more than one observer is performing the study, their watches must be synchronized.

Study Schedule

At least one 10-hour period should be allocated for each pair of approaches studied.  The days generally chosen are Tuesday, Wednesday, or Thursday.  Each study should be a 10-hour counting day extending from 7:30 AM to 12 noon and from 12:45 PM to 6:15 PM.  Variations in these times might be necessary to include peak morning and evening traffic volumes.

Two approach legs are typically observed during the 10-hour survey.  Observations should alternate from one approach to the other approach in 30-minute periods.  Within each 30-minute period, allocate the initial 20 minutes for data gathering and the remaining time for summarizing the data.  This time can also be used to write helpful notations on the forms and to change observation positions.

Data Analysis

A conflict study is used primarily as a diagnostic tool.  The primary objective is to identify predominant conflict types and compare these with crash patterns for the location.  The traffic conflict data can then be used to address safety and operational problems, to recommend corrective measures, or to show the effectiveness of improvements already implemented.

REFERENCES for Conducting Spot Speed Studies and Setting Speed Limits

“Introduction to Traffic Practices – A Guidebook for Local Agencies,” 2nd Edition, Technology Transfer Assistance Program, Missouri Highway and Transportation Department, 1994.

“Manual of Traffic Engineering Studies,” 4th Edition, Institute of Transportation Engineers, 1976.

“Manual of Transportation Engineering Studies,” Institute of Transportation Engineers, 1994.

“Traffic Control Devices Handbook,” Federal Highway Administration, 1983.

REFERENCES for Intersection Sight Distance Studies

“Local Highway Safety Studies-User Guide,” Federal Highway Administration, July 1986.

- “Manual on Uniform Traffic Control Devices for Streets and Highways,” Federal Highway Administration, 1988.

- “A Policy on Geometric Design of Highways and Streets,” American Association of State Highway and Transportation Officials, 1990 (U.S. units) or 1994 (S.I./metric units).

“Traffic Control Devices Handbook,” Federal Highway Administration, 1983.

REFERENCES for Traffic Conflict Studies

Glauz, W. and D. Migletz, “Application of Traffic Conflict Analysis at Intersections,” Transportation Research Board, NCHRP Report 219, 1980.

Parker, M. and C. Zeeger, “Traffic Conflict Techniques for Safety and Operations – Engineers Guide,” Federal Highway Administration, Report No. FHWA-IP-88-026, January 1989.

Parker, M. and C. Zeeger,” Traffic Conflict Techniques for Safety and Operations – Engineers Guide,” Federal Highway Administration, Report No. FHWA-IP-88-027, January 1989.

APPENDIX D

GENERAL GUIDELINES FOR SEVERAL TRAFFIC SAFETY IMPROVEMENTS

Once a location has been identified as needing improvement, it is necessary to select a countermeasure that will achieve the desired results.  In this appendix, the following areas will be discussed:

- Consistency in implementing countermeasures,

- Definitions of warrants, guidelines and crash reduction factors. (They can help determine which countermeasure should be used.), and

- General guidelines for common traffic safety improvements. (The guidelines are not intended to be a substitute for a thorough evaluation of any possible improvements at high-crash locations.)

CONSISTENCY

Be cautious when making a change in the driving environment.  Sometimes, all that is needed to alleviate a traffic problem is a localized, or “spot”, improvement.  Spot improvements will often improve a hazardous location by removing a non-standard roadway element or traffic control device.

All countermeasures should be applied consistently according to the Manual on Uniform Traffic Control Devices (MUTCD) standards so that motorists will have no difficulty in navigating the roadways.  The MUTCD must remain the standard by which traffic control devices are selected, installed, and operated.  The use of non-standard control devices or improvements is not an acceptable practice.

DEFINITIONS

Warrants

Warrants are specific criteria found in the MUTCD.  Generally speaking, warrants must be followed when deciding which traffic control devices or safety improvements to use.  They are based on factors such as crash experience and traffic volume, among others.  A commonly used warrant in the MUTCD is for the installation of devices such as traffic signals.

Warrants are very important since they represent thresholds generally accepted by practicing professionals for the use of specific improvements.  However, the MUTCD is careful to point out that warrants need to be applied with engineering judgment.  Warrants are standards for traffic control device installation, but they do not constitute a legal requirement for installation.

Guidelines

Guidelines usually pertain to situations where selecting countermeasures requires substantial engineering judgment.  Guidelines are based on instances where specific improvements have proven beneficial to motorists and cost-effective to the community.

Several guidelines in this appendix contain suggested thresholds for improvements based on crash experience.  However, remember that the crash experience at a site is due to many factors, and any improvement being considered is only one of many that could be implemented.

An economic analysis should be performed to determine the feasibility of a potential improvement.

Crash Reduction Factors

The guidelines for access control in this appendix, as well as in Appendix G of this manual, contain crash reduction factors.  Crash reduction factors are used to estimate the change in crash experience to be expected from installing a specific improvement.

Most crash reduction factors listed in the HAL Manual are based on studies of improvements at high-crash locations.  Therefore, it is unlikely that there will be any significant reduction in the crash experience at a location if the given location does not have an unusually high crash experience.

general guidelines

Access Control Improvements

There are several points to consider when addressing roadway access control.

Development and the Increased Risk of Crashes

As the traffic volume on a street or highway increases, the neighboring land becomes more attractive to businesses.  Every business needs access to the roadway, but often the driveways are poorly spaced and inadequately designed for the needs of the growing community.  This inevitably leads to traffic delays, disruption in the flow of traffic and crashes, especially rear-end collisions and left-turn crashes.  These problems only increase in severity as more businesses are added and the volume of traffic grows.

Solutions

The solutions to this problem come in different forms.  One possibility is to by-pass the congested area by building another road.  Often this is ruled out because it tends to be expensive and complicated.  The preferred solution is to control access, or to control where vehicles can enter and exit a roadway.  This involves improvements both to the roadway and to driveways.  Some examples of ways to control access include:

- Roadway Improvements

- Left-turn channelization

- Two-way left-turn lane

- Median barriers

- Driveway Improvements

- Widening driveways

- Conversion to one-way driveways

- Combining driveways

- Improving traffic control at driveways

Considering Locations for Improvement

Tables D-1 and D-2 contain minimum crash rates and numbers which, if exceeded, would justify a detailed review of crash data and possible route or spot improvements.  If the existing roadway and driveway volumes are high, or if the crash experience is high at a particular driveway, the MUTCD warrants for traffic signal installation should also be reviewed.  But, while traffic volumes and crash levels indicate the need for access improvements, they should not be the only criteria.  Each roadway, or specific location, must be evaluated with regard to:

- Highway function,

- Traffic speeds,

- Placement of driveways relative to each other,

- Available sight distances, and

- Crash levels.

Crash Reduction

Table D-3 shows the crash reduction expected from several types of access control improvements.  It describes the countermeasure, its general effects, and the crash reduction that may be anticipated.  Table D-3 clearly shows that the crash reduction factors for access improvements vary widely, depending on the traffic volumes and driveway density involved.  A detailed discussion of these improvements, as well as several other types of improvements, is available in the references cited at the end of this appendix.

TABLE D-1:  ACCESS CONTROL:  CRASH THRESHOLDS FOR ROUTE IMPROVEMENTS

TABLE D-2:  ACCESS CONTROL:  CRASH THRESHOLDS FOR DRIVEWAY IMPROVEMENTS

TABLE D-3:  CRASH REDUCTION ESTImATES FOR ACCESS CONTROL AND CHANNELIZATION COUNTERMEASURES

Flashing Beacons

A flashing beacon is a traffic control device used to supplement other devices at potentially hazardous sites.  Flashing beacons consist of one or more sections of a standard traffic signal head with a flashing circular yellow or circular red light in each section.  The MUTCD describes the following types of flashing beacons:

- Hazard Identification Beacon,

- Speed Limit Sign Beacon,

- Stop Sign Beacon, and

- Intersection Control Beacon.

Hazard Identification Beacon

Description:

- A hazard identification beacon flashes yellow.  It should be used only to supplement an appropriate warning or regulatory sign or marker.

Guidelines:

- Use where obstructions are in or immediately adjacent to the roadway.

- Use as a supplement to advance warning signs.

- Use at mid-block crosswalks.

- Use at intersections where warning is required.

- Use to supplement certain regulatory signs.

Speed Limit Sign Beacon

Description:

- A speed limit sign beacon flashes yellow and is used with either a fixed or variable speed limit sign.

Guidelines:

- Use with a speed limit sign to emphasize that the speed limit shown on the sign is in effect.

Stop Sign Beacon

Description:

- A stop sign beacon flashes red and is mounted above the stop sign.

Guidelines:

- Use in locations where surrounding developments and/or commercial lights divert motorists' attention away from the stop sign.

- Use in locations where a stop sign is not immediately visible to the approaching driver due to vertical or horizontal roadway alignment.

Intersection Control Beacon

Examples of Intersection Control Beacons:

- 4-way stop  Beacon flashes red to all approaches.

- 2-way stop  Beacon flashes red to the minor approaches and yellow to the major approaches.

Guidelines:

- Intersection control beacons are intended for use at intersections where volumes or physical conditions do not yet justify conventional traffic signals, but where high crash rates indicate a special hazard exists.  Specifically,

- Four or more left-turn plus right angle crashes occur in one year.  (“Evaluation” 1967)

- Six or more left-turn plus right angle crashes occur in two years.  (“Evaluation” 1967)

Note that the MUTCD does not state warrants for use of an intersection control beacon.

Recommendations for Installation:

- An intersection control beacon should be suspended over the center of an intersection so it is visible from all approaches.

- 2-way stop  Entering volume of the minor road divided by the entering volume of the major road equals 0.50 or less.

- 4-way stop  Entering volume of the minor road divided by the entering volume of the major road is greater than 0.50.

- Installation of a flashing beacon at an offset, multi-leg or "Y" intersection should be avoided since these designs frequently do not provide an adequate line of sight from the driver to the center-mounted flashing beacon.  (Hammer and Tye 1987)

- The driver stopped on the red-controlled approach of a red-yellow beacon may not be aware that drivers on the yellow-controlled approaches do not have to stop. To alleviate this confusion, a supplementary sign may have to be mounted on the minor approach stating that the crossroad traffic does not stop.  (Hammer and Tye 1987)

- An intersection control beacon should be installed only after a proper traffic engineering study has been performed.  This service may be requested through your nearest MoDOT District Office as a part of the Traffic Engineering Assistance Program.

Left-Turn Channelization

Channelization on streets and highways guides drivers through a location.  For either an intersection or a driveway entrance, channelization involves the application of pavement markings or the construction of raised curbs and traffic islands. The two applications for left-turn channelization most commonly used in smaller communities are:

- Providing left-turn lanes on intersection approaches, and

- Constructing a continuous two-way left-turn lane in the middle of a street with numerous driveways.

Each location being considered for a channelization project should be carefully studied before beginning installation to be certain that all traffic islands or markings will safely accommodate vehicles.  This is especially important where it is necessary to provide adequate paths for turns by large vehicles.  A channelization design can be field-tested before permanent installation by temporarily placing sandbags on the roadway to represent curbs or pavement markings.

Left-Turn Lanes

Guidelines for installing a left-turn lane:

- Left-turn lane construction should be considered for intersections having a substantial number of left-turn-involved crashes.  The exact number of left-turn-related collisions justifying a left-turn lane varies depending on several factors.  One of those factors is the occurrence of injury or fatal crashes.

- The criteria listed in Table D-4 are appropriate for considering left-turn lane installation.

Examples of left-turn-involved crashes include:

- Rear-end collisions with vehicles waiting to turn left,

- Same direction sideswipe collisions, and

- Left-turn angle collisions.

Advantages of a left-turn lane include:

- The left-turn lane removes a vehicle from the through lane as it waits for an opportunity to turn.  This separation significantly reduces the danger of rear-end and sideswipe collisions.

- Since opposing, left-turning drivers will be in a direct line with each other, it is easier for them to see opposing through-traffic.

TABLE D-4:  MINIMUM CRASH EXPERIENCE FOR LEFT-TURN LANE CONSIDERATION

Continuous Two-Way Left-Turn Lanes (CTWLTL)

Major two-lane and four-lane urban streets attract a large amount of commercial development along the roadside.  With that development comes an increase in mid-block crashes.  The seriousness of this crash problem usually depends on the number of driveways present, the volume and composition of traffic, and the volume of traffic using the driveways.  An effective countermeasure for reducing these mid-block crashes is to modify the roadway by adding a single lane in the middle known as a continuous two-way left-turn lane (CTWLTL).  Thus, a two-lane road becomes a three-lane road, and a four-lane road becomes a five-lane road.

Guidelines for Installing a CTWLTL:

- If a two-lane undivided or four-lane undivided roadway has a crash rate higher than those listed in Table D-5, the CTWLTL installation should be considered.

- Exact guidelines for when to consider such a major street modification as a CTWLTL are not currently available.  (However, an example of estimated crash rates along commercially developed streets is shown in Table D-5.)

Instructions for Using a CTWLTL:

- A CTWLTL extends for at least several blocks, and it must have signs and markings (see MUTCD) permitting median lane use for left-turns only.

Advantages of a CTWLTL:

- Improves safety for vehicles turning left to enter and exit driveways;

- Separates vehicles traveling in opposite directions, thus reducing the chance for head-on collisions and opposite direction sideswipe collisions;

- Results in fewer delays at driveways;

- Reduces the number of serious mid-block crashes on the through lanes of the street;

- Reduces the number of rear-end collisions and sideswipes due to vehicles waiting to turn left into a drive; and

- Decreases the chance impatient drivers will force their way across oncoming traffic.

TABLE D-5:  TYPICAL ANNUAL CRASH RATES PER MILE FOR NON-INTERSECTION CRASHES IN URBAN COMMERCIAL AREAS  *

Safety Lighting

The primary purpose of roadway lighting, or illumination, is to increase the visibility of the pavement and its surroundings, thereby giving the driver a chance to avoid potentially hazardous situations.  Many studies have stated that the installation of roadway lighting increases safety.

Several suggested warrants for intersection lighting were evaluated in an extensive study of minor safety improvements (Tamburri et al. 1968).  According to this study, “It is recommended that safety lighting be considered at locations which experience 4 night crashes in one year or 6 or more night crashes in two years.”

This study also found that the intersection crashes most susceptible to correction by lighting were single-vehicle crashes (primarily those where a driver proceeded straight at a three-leg intersection on the dead-end leg) and crossing (right-angle) collisions at a four-leg intersection.

A general assumption, which could be applied when evaluating almost any safety lighting project, is that the rate for nighttime crashes should be about equal to the rate for daytime crashes.  The ideal situation would be a ratio of 1.0:1; that is, the crash rate at night is the same as the crash rate during daylight conditions.

Using the decision criteria developed by Walton and Rowan (1974), a ratio of nighttime crashes to daytime crashes of 1.5:1 is somewhat high, but not unusual.  However, a ratio of 2.0:1 or greater indicates that nighttime visibility is inadequate and lighting should be considered for the location.

One-Way Streets

It has been consistently shown that proper planning and implementation of a conversion from two-way streets to one-way streets will reduce total crashes by as much as 10% to 50% on the affected streets.  The crash types that generally see the greatest reduction are:

- sideswipe crashes with vehicles travelling in opposite directions,

- head-on collisions,

- parking crashes,

- right-angle collisions,

- rear-end collisions,

- turning collisions,

- pedestrian crashes, and

- fatal or injury crashes.

Generally, two-way streets should be changed to one-way operation when the following conditions are satisfied:

- There is the possibility of noticeably improving safety along an entire corridor.  (Conversion to one-way streets is not likely to be advantageous if only one or two intersections along a particular street are in the high crash category.)

- It is clear that a specific traffic problem will be alleviated and overall efficiency of the street system will be improved.

- One-way operation is more desirable and cost-effective than the alternative solutions.

- A parallel street of suitable width, preferably not more than a block away, exists or can be constructed.

- The parallel and adjacent streets are continuous in that they carry traffic through and beyond the congested areas.

- A sufficient number of intersecting streets of satisfactory design to permit circulation of traffic exist.

- Safe transition to two-way operation can be provided at the end points of the one-way sections.

- Proper public transit services can continue to be provided on the one-way pair of streets.

- The proposed one-way streets are compatible with the community master plan and adjacent land uses.

- Thorough study shows the advantages of the one-way street system far outweigh the total disadvantages.

Conversion to one-way operation usually involves many intersections and a variety of mid-block situations such as parking, loading zones, alleys, driveways, and pedestrian crossings.  Business owners along a proposed one-way pair of streets are sometimes reluctant to support such an extensive modification in traffic flow as the one-way conversion.  However, the traffic safety improvements and reduced congestion can usually be accomplished without adverse financial impact on adjacent businesses.

Advantages of One-Way Streets

- Capacity is increased by reducing conflicts and by running traffic control devices more efficiently.

- Travel speed is increased as a result of fewer conflicts and delays caused by turning vehicles.  An increase in the number of lanes in one direction also permits easier passing of slower or double-parked vehicles.

- One-way operation permits good progressive timing of signals.

- The number and severity of crashes is reduced by eliminating head-on crashes and reducing several types of intersection conflicts.

- Full use can be made of an odd number of traffic lanes when traffic flows in only one direction.  When a street is used in two directions, fewer lanes may be possible due to width requirements.

- On-street parking that would have otherwise been removed might be retained due to better use of the street width.

Disadvantages of One-Way Streets

- Travel distances to certain destinations may be increased by having to drive around the block.

- One-way streets may be confusing to strangers.

- Emergency vehicles may be blocked at intersections by vehicles waiting in all lanes on an approach.

- Additional signs and markings must be installed and must be carefully maintained (see MUTCD).

A possible change to one-way streets should be thoroughly evaluated with the assistance of traffic engineering professionals.  It is possible for Missouri communities that do not have a traffic engineer on staff to arrange for these services through the MoDOT Traffic Engineering Assistance Program.

Roadside Safety Features

When a moving vehicle unintentionally leaves the roadway, overturning or collision with a fixed object is likely to occur unless a safe roadside has been provided.  Two characteristics of the roadside generally determine whether a vehicle will recover safely after leaving the roadway: the roadside geometry and the presence of non-yielding large objects.

Roadside Geometry

Roadway embankments are classified as recoverable, non-recoverable, or critical.

- Recoverable slopes:		4:1 (horizontal to vertical) or flatter

A motorist who encroaches on a recoverable slope can usually regain control of the vehicle if no hazardous objects are encountered.

- Non-recoverable slopes:	range from 4:1 to as steep as 3:1

Motorists on side slopes this steep usually are not able to stop the vehicle until it travels to the bottom of the embankment.

- Critical slopes:		greater than 3:1

A vehicle is most likely to overturn on a critical slope.

If a critical embankment exists along an urban street, a barrier such as a guardrail should intercept errant vehicles before reaching the side slope.  The height of the embankment is related to the necessity for the barrier as shown in Figure D-1.

Roadside Obstacles (Fixed Objects)

Roadside obstacles may be non-traversable hazards or fixed objects.  Ideally, a reasonable recovery area, or “clear zone,” containing no hazards should be provided along the roadway.  Alternatives for dealing with existing roadside hazards are usually considered in this order:

- Remove the obstacle or redesign it so it can be safely traversed.

- Re-locate the obstacle so it is less likely to be struck.

- Reduce impact severity by using a breakaway device for signs and light poles.

- Re-direct a vehicle by installing a barrier or crash cushion.

- Delineate the obstacle if above alternatives are not appropriate.

Figure D-1:  Comparative risk warrants for embankments (adapted from aashto roadside design guide 1996)

Alternative 1 & Alternative 2:

Removing or redesigning an object is highly preferred, but it is not always practical in urban areas.

Signs, signals, and light poles must be located near the road in most cities.  This practice often makes it difficult to increase safety at the side of the road.

Alternative 3:

Breakaway devices are easily provided, and they are extremely effective in reducing vehicle occupant injuries.  It may be possible to bury a utility line and thereby eliminate an entire series of poles.

Alternative 4:

Installing a barrier requires consideration of applicable warrants (e.g. guardrail).

A barrier should be installed only if it is apparent that the results from a vehicle striking the barrier will be less severe than the crash resulting from hitting the unshielded object.  Although no specific number of crashes may be related to the need for installing a barrier, general guidelines do exist for their use, as shown in Table D-6.  When a barrier is installed, the following things should be considered:

Design:	Specific roadside barrier designs depend on the function the barrier must perform, as well as the speed and size of the involved vehicle.

Location:	The barrier should be placed as far from the traveled way as conditions permit.

Size:	The length of barrier must be determined based on the length of the hazard and the vehicle approach path.

Lateral Offset:	The lateral offset of the barrier from the fixed object must be sufficient to allow for barrier deflection.

Alternative 5:

Delineating the obstacle alerts the motorist to presence of hazardous objects.

Hazardous objects can be delineated using markers recommended in the MUTCD (Section 3C).  Types of roadside hazards especially prevalent in urban areas include trees, mailboxes, and drainage features.

Table d-6:  guidelines for roadside barriers                                      (AASHTO Roadside Design Guide 1996)

Trees

A tree with a trunk diameter greater than 6 inches is considered a fixed object.  The recommended distance of trees from a roadway depends on the design speed of the road, as shown in Table D-7.

| Design Speed | Minimum Setback from Edge of Road |
| --- | --- |
| 50 mph or more | 30 feet |
| Less than 45 mph | 7 – 18 feet |

TABLE D-7:  RECOMMENDED SPACING OF TREES FROM ROADWAY

If these distances are impractical for a community, the removal of trees should be prioritized according to the danger they present.  For instance, trees located along curves are a greater hazard than trees along straight sections.

Mailboxes

Roadside mailbox installations result in an object being placed very close to the traveled path, with the mailbox typically at the height of a vehicle's windshield.

- Mailbox supports should be a nominal 4-inch by 4-inch wood post, or metal post with strength no greater than a 2-inch diameter standard strength steel pipe, embedded no more than 24 inches.

- Mailbox-to-post attachments should prevent mailboxes from separating from their supports when hit by an errant vehicle.

Drainage Features

Culverts, inlets, headwalls, and ditches are serious traffic hazards if they are not properly designed and located.  The following guidelines pertain to drainage structures:

- Eliminate non-essential drainage structures.

- Design or modify drainage structures so they are traversable or present a minimal hazard to an errant vehicle.

- If a major drainage feature cannot be re-designed or re-located, it should be shielded by a suitable traffic barrier.

- Roadside hardware, such as posts, should not be in or near a ditch bottom.

- Drop inlets on the roadway should be installed flush with the pavement surface and designed for safe passage of bicycle tires.

- Drop inlets located off the traveled way should be installed flush with the ditch bottom or slope on which they are located.

TRAFFIC CONTROL AT LOW-VOLUME INTERSECTIONS

A community should adopt a signing policy for low-volume intersections that can be applied with a high degree of consistency throughout the jurisdiction.  This policy should not be unnecessarily restrictive.  In particular, installation of unnecessary stop signs must be avoided since this will cause drivers to develop disrespect for all stop signs.

The decision to provide yield signs or stop signs, rather than using no control at a low volume intersection, is based on:

- Sight distances,

- Traffic volumes,

- Vehicle speeds on the approaches,

- Crash experience at the site, and

- Benefits from protecting traffic on designated through streets.

The AASHTO procedures for evaluating intersection sight distances and safe approach speeds must always be used when selecting the type of signs to install at a low volume intersection (refer to Appendix C, HAL Manual).  With respect to intersection control, the MUTCD does not contain specific volume and/or crash warrants for yield signs or stop signs, except for multi-way stop signs.

No Control at Intersections

Guidelines:

- Both streets are local streets; or

- One street is a local street and the other is a minor collector; and

- Volume does not exceed 2,000 vehicles per day on the busiest roadway.

DO NOT:

- Use an un-controlled intersection if the busiest roadway has a volume greater than 2,000 vehicles per day.

Comment:  Many intersections operating with no control have such low volumes that very few crashes occur, perhaps only one crash every three years.  The occurrence of this one crash does not necessarily justify installing yield signs or stop signs.  Refer to the following guidelines to determine whether yield signs or stop signs should be installed at a particular intersection.  It can also be helpful to consult other sources, such as the AASHTO “Policy on Geometric Design of Highways and Streets”.

Yield Signs at Intersections

Guidelines:

- Three or more crashes occur during three years involving vehicles on the minor road; or

- Two or more crashes occur in one year with vehicles on the minor road.

DO NOT:

- Use yield signs to regulate the major traffic flow at an intersection.

Special Instructions for Installation:

- Make sure only the motorists required to yield can view the yield sign.  This is especially important if yield signs are used where two roadways meet at an acute angle.  Install the signs at an angle or shield the lettering.

Two-Way Stop Signs at Intersections

Guidelines:

- Four or more crashes occur during three years involving vehicles on the minor road; or

- Three or more crashes occur in one year involving vehicles on the minor road.

DO NOT:

- Use a two-way stop sign to regulate the major flow at an intersection.

- Use a stop sign to control speed along a street.

- Use a portable stop sign except for emergency purposes.

Special Instructions for Installation:

- Before installing, complete an on-site field report to determine if some other less restrictive countermeasures could be implemented.

- Make sure only the motorists required to stop can view the stop sign.  This is especially important if two-way stop signs are used where two roadways meet at an acute angle.  Install the signs at an angle or shield the lettering.

Multi-way (Three-Way or Four-Way) Stop Signs at Intersections

Guidelines:

- Intersection has five or more correctable crashes in one year.  (Correctable crashes include right-turn collisions, left-turn collisions, and right-angle collisions.)

- Traffic volumes on all approaches are about equal.

- Traffic volumes are high.  (In the case of high traffic volumes, a traffic volume study should be performed to determine if the MUTCD traffic signal warrants have been met.)

- Sight distances at the intersection are inadequate.

Before Installing:

- Evaluate other countermeasures (improving skid resistance, restricting parking at the intersection, e.g.).

- Conduct a traffic volume study if the volume of traffic seems to be high.

REFERENCES for ACCESS CONTROL IMPROVEMENTS

Flora, J. and K. Keitt, “Access Management for Streets and Highways,” Federal Highway Administration, Report No. FHWA-IP-82-3, June 1982.

Glennon, J., et al., “Technical Guidelines for the Control of Direct Access to Arterial Highways,” Federal Highway Administration, Report Nos. FHWA-RD-76-85 through 87, August 1975.

“Guidelines for Driveway Design and Location,” Institute of Transportation Engineers, 1985.

Marks, H., “Protection of Highway Utility,” Transportation Research Board, NCHRP Report 121, 1971.

Stover, V., et al., “Guidelines for Medial and Marginal Access Control on Major Roadways,” Transportation Research Board, NCHRP Report 93, 1970.

Stover, V. and F. Koepke, “Transportation and Land Development,” Institute of Transportation Engineers, 1988.

REFERENCES for FLASHING BEACONS

“Evaluation of Minor Improvements: Part 1 - Flashing Beacons,” Traffic Department, State of California Transportation Agency, 2nd Ed., 1967.

J. Hammer and E. Tye, “Overhead Yellow-Red Flashing Beacons,” Division of Traffic Engineering, California Department of Transportation, Report No. FHWA/CA/TE-87/01, 1987.

“Manual on Uniform Traffic Control Devices for Streets and Highways,” Federal Highway Administration, 1988.

“Synthesis of Safety Research Related to Traffic Control and Roadway Elements,” FHWA-TS-82-232, Federal Highway Administration, 1982.

REFERENCES for LEFT TURN CHANNELIZATION

“Accident Reduction Factors for Highway Safety Projects,” in Safety Evaluation Instructions, California Department of Transportation, 1975.

“Design Criteria for Left-Turn Channelization,” Technical Council Informational Report, Institute of Transportation Engineers, ITE Journal, February 1981, pp. 38-43.

Harwood, D., “Multilane Design Alternatives for Improving Suburban Highways,” Transportation Research Board, NCHRP Report 282, 1986.

“Manual on Uniform Traffic Control Devices for Streets and Highways,” Federal Highway Administration, Washington, D. C., 1988.

Neuman, T., “Intersection Channelization Design Guide,” Transportation Research Board, NCHRP Report 279, 1985.

REFERENCES for SAFETY LIGHTING

“Roadway Lighting Handbook,” Federal Highway Administration, Implementation Package 78-15, December 1978 and Addendum September, 1983.

“Synthesis of Safety Research Related to Traffic Control and Roadway Elements,” FHWA-TS-82-233, Federal Highway Administration, 1982.

Tamburri, T., et al., “Evaluation of Minor Improvements,” Highway Research Board, Highway Research Record Number 257, 1968.

Walton, N. and N. Rowan, “Warrants for Highway Lighting,” Transportation Research Board, NCHRP Report 152, 1974.

REFERENCES for ONE-WAY STREETS

“A Policy on Geometric Design of Highways and Streets,” American Association of State Highway and Transportation Officials, 1990 (U.S. customary units) or 1994 (S.I. units).

“Synthesis of Safety Research Related to Traffic Control and Roadway Elements,” FHWA-TS-82-232, Federal Highway Administration, 1982.

“Transportation and Traffic Engineering Handbook,” 2nd Edition, Institute of Transportation Engineers, Prentice-Hall, Inc., 1982.

REFERENCES for ROADSIDE SAFETY FEATURES

“A Guide for Accommodating Utilities Within Highway Right-of-Way,” American Association of State Highway and Transportation Officials, 1985.

“A Guide for Erecting Mailboxes on Highways,” American Association of State Highway and Transportation Officials, 1984.

“Guide to Management of Roadside Trees,” FHWA-IP-86-17, Federal Highway Administration, December 1986.

“Manual on Uniform Traffic Control Devices for Streets and Highways,” Federal Highway Administration, 1988.

“Roadside Design Guide,” American Association of State Highway and Transportation Officials, 1996.

“Traffic-Safe and Hydraulically Efficient Drainage Practice,” Highway Research Board, NCHRP Synthesis of Highway Practice, No. 3, 1969.

REFERENCES for INTERSECTION CONTROL

“A Policy on Geometric Design of Highways and Streets,” American Association of State Highway and Transportation Officials, 1990 (U.S. customary units) or 1994 (S.I. units).

“Manual on Uniform Traffic Control Devices for Streets and Highways,” Federal Highway Administration, 1988.

McGee, H. and M. Blankenship, “Guidelines for Converting Stop to Yield Control at Intersections,” Transportation Research Board, NCHRP Report 320, 1989.

Stockton, W, et al., “Stop, Yield, and No Control at Intersections,” Federal Highway Administration, Report No. FHWA-RD-81/084, June 1981.

APPENDIX E

ESTIMATED IMPROVEMENT PROJECT COSTS – 1999

The roadway and traffic improvement cost estimates provided below were obtained from the Missouri Department of Transportation and are current for the year 1999.  It is possible that local costs could vary from those listed below due to the location and/or project size.  Unless otherwise noted, the costs are for installation (materials and labor) only.  To account for additional overhead and administrative costs it is suggested that the initial cost of a project be increased by about 30%, or by the percentage deemed appropriate for the jurisdiction.

| IMPROVEMENT DESCRIPTION |  | 1999 COST |
| --- | --- | --- |

ROADWAY CONSTRUCTION/RECONSTRUCTION

Roadway grading and paving (widening)	            $   3.80	SF*

Roadway grading and paving (reconstruction)	4.66 	SF

Median construction (concrete, excluding curbing)	3.50 	SF

Curb and gutter (barrier and mountable)	12.50 	LF

Barrier curbing	19.00 	LF

Shoulder Construction (6” gravel)	4.15 	SY

Curb removal	3.50 	LF

Curb inlet	443.00 	EA

Driveway closure; new curbing installation	19.65 	SY

Driveway construction	55.00 	SY

Island construction (concrete, excluding curbing)	3.50 	SF

PAVEMENT SURFACE TREATMENTS

Overlay (1-1/2” thick; lime/steel/slag)	1.40 	SY

Chip and seal (3/4” thick; with special rock gradation)	1.30 	SY

Slurry seal (special stone gradation in suspension)	2.00 	SY

Pavement grooving	1.50 	SF

Pavement striping (4-inch white or yellow stripe)	0.10 	LF

Pavement marking (stop bars, lane use arrows, etc.)	3.50 	SF

TRAFFIC SIGNALS AND BEACONS

Overhead 4-way flashing beacon	     2,000.00 	EA

Post, signal, 10 feet high	430.00 	EA

Mast arm post	2,750.00 	EA

Fixed-time controller	5,000.00 	EA

Actuated Controller	7,500.000 	EA

Junction box	250.00	EA

Detector, loop inductive	3000 	EA

Detector, magnetic	371.00 	EA

| IMPROVEMENT DESCRIPTION |  | 1998 COST |
| --- | --- | --- |

Detector, pedestrian pushbutton	$  132.00 	EA

Conduit (pushed . . . 2-inch diameter)	25.00 	LF

Conduit (trenched . . . 2-inch diameter)	11.00 	LF

ROADSIDE FEATURES

Guardrail:	New (Type A)	12.00	LF

Breakaway Cable Terminal (BCT)	700.00 	EA

Bridge attachment	700.00 	EA

Guardrail, New (Type A) and remove previous guardrail	20.00 	LF

Complete lighting unit (1 Pole)	1,600.00 	EA

Steel breakaway sign post	25.00 	LF

Wood sign post (4-inch by 4-inch)	1.00 	LF

Sign (installed . . . stop, yield, warning, etc.)	112.00 	EA

Delineators (installed . . . sign and post)	60.00 	EA

Remove and reset wood utility pole	160.00 to 750.00 	EA

Remove and reset wood telephone poles	330.00 to 4,000.00 	EA

Remove and reset road sign and post	50.00 	EA

Remove tree(s)	100.00 to 650.00 	EA

RAILROAD GRADE CROSSINGS

Railroad crossing surface improvement (1 track)

Asphalt		200.00 	LF

Concrete	400.00 	LF

Timber		300.00 	LF
		Rubberized	500.00 	LF

Railroad crossing automatic gates (per crossing)	100,000.00	TYP

Railroad crossing flashing lights (per crossing)	80,000.00 	TYP

MISCELLANEOUS

Sidewalk removal	4.00 	SY

## Sidewalk construction	3.41 	SF

Sodding		3.41 	SY

Blade gravel road approaches (4) at intersection	250.00 	TYP

THE FOLLOWING ITEMS INCLUDE MATERIAL COST ONLY:

Plastic three-lens signal head (12-inch lenses)	195.00	EA

Plastic two-lens pedestrian head (12-inch lenses)	160.00 	EA

Optically programmed three-lens signal head	750.00 	EA

Plastic back-plate for three-lens signal head	70.00 	EA

*Unit Cost Symbols:	EA	=	Each

LF 	= 	Lineal Foot

SF 	= 	Square Foot

SY 	=	Square Yard

TYP 	= 	Typical

APPENDIX F

ESTIMATED IMPROVEMENT PROJECT SERVICE LIFE

The estimated improvement project service lives listed below were obtained from the Missouri Department of Transportation and two other state highway agencies.  It should be noted that the service life of an improvement project is somewhat difficult to forecast for several reasons, such as the quality of maintenance the project will receive.  Local estimates should be used for service lives whenever they are available.  However, there is very little benefit to be gained in stating service lives of an unusual number of years, such as 14 years or 29 years.  Such estimates do not have much credibility, and they can make the economic analysis more complicated.

|  |  | SERVICE LIFE |
| --- | --- | --- |
| IMPROVEMENT DESCRIPTION |  | (Years) |

ROADWAY CONSTRUCTION/RECONSTRUCTION

Widen pavement, no lanes added	20

Add lanes, no new median	20

Divide highway, add new median	20

Widen or improve shoulder	10

Flatten, clear side slopes	20

Relocate driveways	20

Flatten entrance slopes	20

Acquire right-of-way	100

Change horizontal alignment	15

Change vertical alignment	15

Change horizontal and vertical	15

STRUCTURES CONSTRUCTION/RECONSTRUCTION

Widen bridge or major structure	20

Replace bridge or major structure	30

Construct new bridge or major structure	30

Construct minor structure	20

Construct pedestrian over- or under-crossing	30

Construct interchange	35

PAVEMENT SURFACE TREATEMENTS

Apply skid treatment, groove pavement	10

Apply skid treatment, overlay pavement	6-9

Apply skid treatment, seal coat	3-5

|  |  | SERVICE LIFE |
| --- | --- | --- |
| IMPROVEMENT DESCRIPTION |  | (Years) |

PAVEMENT SURFACE TREATEMENTS (cont’d)

Apply skid treatment, slurry seal	5-7

Apply markings (paint)	1

Apply markings (thermoplastic)	5

Apply edge-line markings (paint)	2

ROADSIDE FEATURES

Install illumination	15

Install breakaway sign support	10

Install breakaway luminaire support	20

Install guardrail	10

Install median barrier	15

Improve drainage structures	20

Install fencing	10

Install traffic signs	6-8

INTERSECTION-RELATED PROJECTS

Channelize, add turning lanes	15

Traffic signals	15

Warning flashers	15

Illumination	15

Overhead flashing beacon	10

RAILROAD GRADE CROSSINGS

Grade separation	30

Crossing relocation	30

Crossing illumination	15

Automatic gates	20

Flashing lights	20

Crossing signs and markings	5

Crossing surface improvement

Asphalt-timber	10

Timber	5

Rubberized	15

Concrete	20

OTHER IMPROVEMENTS

Delineators	10

Raised pavement markers	5

Improve sight distance	10  (variable)

REFERENCES FOR SERVICE LIFE ESTIMATES

- J. McCoy, “Safety Improvement Economic Analysis,” Iowa Department of Transportation, Memo Reference No. 590, November 27, 1985.

Missouri Highway and Transportation Department, Correspondence dated April 4, 1990, Jefferson City, Missouri.

University of Alabama, “Accident Identification & Surveillance Documentation Manual,” TSM Report 112-88, September 1988.

APPENDIX G

ESTIMATED CRASH REDUCTION FACTORS

The estimated crash reduction (CR) factors in Table G-1 are based on safety project evaluations performed by a variety of groups and agencies throughout the United States.  Due to the variability in traffic crash characteristics and countermeasure effectiveness among sites and regions, differences in CR factors for specific improvements do exist among agencies.  Whenever possible, an agency should monitor its traffic safety improvement projects and develop its own CR factors.

CR factors are required for estimating the economic benefits likely to result from feasible countermeasures.  Each CR factor indicates the percent crash reduction for a single countermeasure.

When applying CR factors, good engineering judgment and common sense must prevail.  It is essential that each CR factor be applied to only those crashes having a reasonable chance of being corrected by the associated countermeasure.

The Estimated Crash Reduction Factor table is organized according to countermeasure category and CR factor group.  The countermeasure categories are printed in capital letters in the left column, and the CR factor groups are identified by Roman numerals at the top of the table.

COUNTERMEASURE CATEGORIES

The countermeasure categories are tabled in the following sequence:

- Channelization

- Construction/Reconstruction

- Traffic Signs

- Traffic Signals

- Illumination

- Pavement Treatment

- Pavement Markings

- Regulations

- Roadside Improvement

- Delineation

Within each major countermeasure category, sub-categories are listed.  For instance, under the category “REGULATIONS” there are sub-categories such as “Regulate On-Street Parking” and “Prohibit Left Turns.”

When several countermeasures are being considered for simultaneous use to correct a crash pattern at one location, the combined effect must be calculated using the procedure in the section entitled “COUNTERMEASURE ANALYSIS” in Chapter 5.  If that procedure is not followed, the crash reduction estimate will be incorrect.

CRASH REDUCTION (CR) FACTOR GROUPS

The CR factors are grouped to provide guidance for their proper application.  The five groups listed across the top of the table are defined as follows:

GROUP I:	Contains CR factors applicable to “All” crashes.

GROUP II:	Contains CR factors applicable to crashes according to severity level, “Fatal/Injury” or “PDO”.

GROUP III:	Contains CR factors applicable to several different types of crashes, such as “Head On” or “Right Angle”.

GROUP IV:	Contains CR factors applicable to crashes that occur during “Wet Pavement” conditions.

GROUP V:	Contains CR factors applicable to crashes that occur during “Night” conditions.

GROUP VI:	Contains CR factors applicable to crashes that are train-related.

It is recommended that, for a specific countermeasure, the CR factor(s) to be applied should be selected from only one of the five groups.  For example, if the countermeasure is “PAVEMENT TREATMENTS – de-slick pavement” for a high-crash intersection, the engineer should choose the most meaningful application of CR factors from these possibilities:

- From Group I:  Apply 13% reduction to All crashes; or

- From Group III:  Apply CR factors to specific crash types, as:  10% reduction to Head On; 40% to Rear End, 10% to Right Angle, 10% to Side-Swipe; 10% to Fixed Object; 10% to Pedestrian, and 10% to Run-Off Road crashes; or

- From Group IV:  Apply 55% reduction to Wet Pavement crashes.

If CR factors are applied from more than one group for the proposed “De-slicking” countermeasure, the crash reduction may be substantially overestimated.  Of course, the ideal situation would be to have CR factors for both wet and dry pavement conditions, for each crash type, and for each level of severity.  However, CR factors are seldom available at that level of detail.

For additional access control measures, see appendix D.  Table D-3 contains information on crash reduction, in a different format, as a function of ADT.

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| #### CHANNELIZATION (see also Table D-3 in Appendix D) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| channelize intersection (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| provide left-turn lane (with | 25 |  |  |  |  |  |  | 45 |  |  |  |  |  |  |  |
| signal) (1, 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - with no left-turn phase | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - existing left-turn phase | 35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| provide left-turn lane (without | 35 |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  |
| signal) (1, 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - painted lane | 32 |  |  |  | 75 |  |  |  |  |  |  |  |  |  |  |
| - protected lane with curb or | 67 |  | 62 |  | 93 |  |  |  |  |  |  |  |  |  |  |
| raised bars |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| provide right-turn lane (1) | 25 |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |
| increase turn lane length (1) | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install two-way left-turn | 35 | 20 | 35 |  | 36 |  |  | 33 |  |  |  | 37 |  |  |  |
| lane in median (2, 8, 28) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - two-lane to three-lane | 32 | 59 |  |  | 46 |  | 46 | 46 |  |  |  |  |  |  |  |
| - four-lane to five-lane | 28 | 42 |  |  | 40 |  | 40 | 40 |  |  |  |  |  |  |  |
| add mountable median (1) | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add non-mountable median (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### CONSTRUCTION/ RECONSTRUCTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REALIGNMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| construct a more gradual | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| horizontal curve (1,12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 20 to 10 degrees | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 15 to 5 degrees | 63 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 10 to 5 degrees | 45 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1:  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| CONSTRUCTION/ RECONSTRUCTION (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REALIGNMENT (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve vertical curve (1) | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve horizontal and | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| vertical curve (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve sight distance at | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| intersection (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SEPARATING DEVICES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| close median opening (3) |  |  |  | 100 | 50 | 100 | 50 | 100 |  |  |  |  |  |  |  |
| install median barrier (1, 2, 5) | 5 | F:65 |  |  |  |  |  |  |  |  |  | 35 |  |  |  |
|  |  | I:40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - install a 1 to 12 ft. median |  | F:75 | -28* |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I:2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - install a 13 to 30 ft. median |  | F:85 | -30* |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I:5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install concrete median |  | F:90 | -10* |  |  |  |  |  |  |  |  |  |  |  |  |
| barrier (5) |  | I:10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install/improve curbing (9) |  |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |
| replace active warning | 95 |  | 88 |  |  |  |  |  |  |  |  |  |  |  |  |
| devices with bridge or tunnel (5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * A crash reduction factor preceded by a (-) sign indicates an increase should be expected for that type of crash. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| CONSTRUCTION/ RECONSTRUCTION (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PAVEMENT WIDENING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| widen pavement (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| widen shoulder (paved) (10) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - widen 2 ft. |  |  |  |  |  |  |  |  |  | 16 |  | 16 |  |  |  |
| - widen 4 ft. |  |  |  |  |  |  |  |  |  | 29 |  | 29 |  |  |  |
| - widen 6 ft. |  |  |  |  |  |  |  |  |  | 40 |  | 40 |  |  |  |
| - widen 8 ft. |  |  |  |  |  |  |  |  |  | 49 |  | 49 |  |  |  |
| widen shoulder (unpaved) (10) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - widen 2 ft. |  |  |  |  |  |  |  |  |  | 13 |  | 13 |  |  |  |
| - widen 4 ft. |  |  |  |  |  |  |  |  |  | 25 |  | 25 |  |  |  |
| - widen 6 ft. |  |  |  |  |  |  |  |  |  | 34 |  | 34 |  |  |  |
| - widen 8 ft. |  |  |  |  |  |  |  |  |  | 43 |  | 43 |  |  |  |
| pave shoulder (1) | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| stabilize shoulder (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| widen lane (10) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - add 1 ft. to both sides |  |  |  | 12 |  |  | 12 |  |  |  |  | 12 |  |  |  |
| - add 2 ft. to both sides |  |  |  | 23 |  |  | 23 |  |  |  |  | 23 |  |  |  |
| - add 3 ft. to both sides |  |  |  | 32 |  |  | 32 |  |  |  |  | 32 |  |  |  |
| - add 4 ft. to both sides |  |  |  | 40 |  |  | 40 |  |  |  |  | 40 |  |  |  |
| ADDITIONAL LANES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add passing/climbing lane (28) | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add accel./decel. lane (1) | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add lanes (2) | 25 | F:39 | 27 | 53 | 32 |  | 30 |  |  |  |  | 44 |  |  |  |
|  |  | I:23 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| CONSTRUCTION/ RECONSTRUCTION (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BRIDGES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| widen bridge (general) (1, 2, 4) | 45 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 18 to 24 ft. | 68 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 20 to 24 ft. | 56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 22 to 24 ft. | 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 18 to 30 ft. | 93 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 20 to 30 ft. | 90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 22 to 30 ft. | 86 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| replace two-lane bridge (1, 2) | 45 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| repair bridge deck (1) | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| INTERSECTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| increase turning radii (1) | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve sight distance (1, 2, 9) | 30 |  |  | 10 |  | 21 | 10 | 13 |  |  | 10 |  |  |  |  |
| PEDESTRIAN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| construct pedestrian bridge | 5 |  |  |  |  |  |  |  |  |  | 90 |  |  |  |  |
| or tunnel (1, 13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install sidewalk (1) |  |  |  |  |  |  |  |  |  |  | 65 |  |  |  |  |
| DRAINAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| provide adequate drainage (1) | 20 |  |  |  |  |  |  |  |  |  |  |  | 40 |  |  |
| provide proper | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| superelevation (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FREEWAY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| construct interchange (1) | 55 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| modify entrance/exit ramp (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| construct frontage road (1) | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install glare screen (1) |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| CONSTRUCTION/ RECONSTRUCTION (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GUARDRAIL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install guardrail (1, 2) | 5 | F:65 |  |  |  |  |  |  |  |  |  | 30 |  |  |  |
|  |  | I:40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| upgrade guardrail (1, 2) | 5 | F:50 |  |  |  |  |  |  |  |  |  | 26 |  |  |  |
|  |  | I:35 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install at bridge (5) |  | F:90 | -110* |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I:45 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install along ditch (5) |  | 26 | -19* |  |  |  |  |  |  |  |  |  |  |  |  |
| install along embankment (5) |  | 42 | -47* |  |  |  |  |  |  |  |  |  |  |  |  |
| install to shield trees (5) |  | F:65 | -90* |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I:51 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install to shield fixed objects as |  | 31 | -45* |  |  |  |  |  |  |  |  |  |  |  |  |
| rocks and steel posts (5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### TRAFFIC SIGNS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WARNING SIGNS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install warning signs (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install warning signs in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| advance of intersections (1, 11) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - urban | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - rural | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install warning signs in | 30 | F:55 |  | 29 |  |  |  |  |  |  |  | 30 |  |  |  |
| advance of curves (1, 2, 11) |  | I:20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add signs at railroad |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 |
| crossings (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install school zone signs (1) | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install pavement condition |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |
| signs (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * A crash reduction factor preceded by a (-) sign indicates an increase should be expected for that type of crash. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| TRAFFIC SIGNS (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REGULATORY SIGNS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install stop sign (2-way) (1) | 35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Change to all-way stop sign from | 55 |  |  |  | 13 | 72 |  | 20 |  |  | 39 |  |  |  |  |
| two-way stop sign (1, 26) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install yield sign (1) | 45 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install lane use signs (27) | 30 |  |  |  | 10 |  | 20 |  |  |  |  |  |  |  |  |
| GUIDE SIGNS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install guide signs (1) | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install variable message | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sign (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### TRAFFIC SIGNALS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install signal (general) (1, 24) | 25 |  |  |  |  | 65 |  |  |  |  |  |  |  |  |  |
| - from two-way stop | 28 | 43 |  |  | -46* | 74 |  | -92* |  |  |  |  |  |  |  |
| - from two-way stop and add | 36 | 53 |  |  | 8 | 74 |  | -43* |  |  |  |  |  |  |  |
| left-turn lane |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIGNAL UPGRADE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| upgrade signal (1) | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install 12-inch lenses (1) | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install visors or back-plates (1) |  |  |  |  |  | 20 |  |  |  |  |  |  |  |  |  |
| install optically programmed | 15 |  |  | 20 | 10 | 10 |  | 10 |  |  |  |  |  |  |  |
| signal lenses (1, 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| upgrade pedestal mounted to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mast arm: pre-timed signal (24) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - no left-turn lane | 51 | 52 |  |  | 24 | 69 |  | 28 |  |  |  |  |  |  |  |
| - existing left-turn lane | 44 | 25 |  |  | 35 | 74 |  | 2 |  |  |  |  |  |  |  |
| - left-turn lane added | 84 | 87 |  |  | 72 | 83 |  | 87 |  |  |  |  |  |  |  |
| * A crash reduction factor preceded by a (-) sign indicates an increase should be expected for that type of crash. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| TRAFFIC SIGNALS (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIGNAL PHASING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve signal phasing (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add exclusive left-turn phase (1) | 25 |  |  |  |  |  |  | 70 |  |  |  |  |  |  |  |
| add protected/permissive | 10 |  |  |  |  |  |  | 40 |  |  |  |  |  |  |  |
| left-turn phase (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve timing (1) | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install/improve pedestrian | 25 |  |  |  |  |  |  |  |  |  | 55 |  |  |  |  |
| signal (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve yellow change interval (1) | 15 |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |
| add all-red interval (1) | 15 |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |
| interconnect signals (1, 15) | 15 | 29 |  |  | 20 | 10 |  | 38 | 36 |  | 10 |  |  |  |  |
| install traffic actuated signal (33) |  |  |  |  |  | 10 | 20 | 80 |  |  |  |  |  |  |  |
| REMOVAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| remove unwarranted signal (1, 9) | 50 |  |  |  | 90 | -30* |  | -10* |  |  | -10* |  |  |  |  |
| FLASHING BEACON |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install flashing beacon (1) | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install flashing beacon at | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| intersection (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install intersection advance | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| warning flashers (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install general advance warning | 35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| flashers (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * A crash reduction factor preceded by a (-) sign indicates an increase should be expected for that type of crash. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| TRAFFIC SIGNALS (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAILROAD CROSSINGS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| general railroad crossings (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 |
| add flashing lights at railroad |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 65 |
| crossings (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add automatic gates at |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 |
| railroad crossings (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add automatic gates and |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 |
| flashing lights (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### ILLUMINATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve street lighting (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  | 50 |  |
| install/improve lighting at | 25 |  |  |  |  |  |  |  |  |  |  |  |  | 45 |  |
| roadway segment (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install/improve lighting at | 30 |  |  |  |  |  |  |  |  |  |  |  |  | 50 |  |
| intersections (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install/improve lighting at | 25 |  |  |  |  |  |  |  |  |  |  |  |  | 50 |  |
| interchanges (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install/improve lighting at | 30 |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 60 |
| railroad crossings (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### PAVEMENT TREATMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| de-slick pavement (9, 21) | 13 |  |  | 10 | 40 | 10 | 10 |  |  | 10 | 10 | 10 | 55 |  |  |
| groove pavement (1) | 25 |  |  |  |  |  |  |  |  |  |  |  | 60 |  |  |
| resurface curve with skid- |  |  |  | 86 |  |  |  |  |  |  |  |  | 51 |  |  |
| resistant overlay (21) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| resurface (general) (1) | 25 |  |  |  |  |  |  |  |  |  |  |  | 45 |  |  |
| install rumble strips (1, 2) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| groove shoulder (1, 2) | 25 | 18 | 17 |  |  |  |  |  |  |  |  | 27 |  |  |  |
| make surface improvements | 34 |  | 39 |  |  |  |  |  |  |  |  |  |  |  |  |
| at railroad crossings (11) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| #### PAVEMENT MARKINGS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add pavement markings (32) | 13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add pavement markings at | 48 | 42 | 51 |  | 58 |  |  |  |  |  |  |  |  |  | 15 |
| railroad crossings (1, 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add reflectorized raised | 10 |  |  | 20 |  |  | 20 |  |  | 10 |  | 10 | 25 | 20 |  |
| pavement markings (1, 9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add "no passing" striping (1) |  |  |  | 40 |  |  | 40 |  |  |  |  |  |  |  |  |
| add centerline markings (1) | 35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add edgeline markings (1, 20) | 15 | 15 | 8 |  |  |  |  |  |  |  |  | 30 |  |  |  |
| add/improve pedestrian |  |  |  |  |  |  |  |  |  |  | 25 |  |  |  |  |
| crosswalk (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add wider markings (1) |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 |  |
| #### REGULATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| prohibit on-street parking (1, 9) | 35 |  |  |  | 10 | 10 | 30 |  |  | 40 | 30 |  |  |  |  |
| change angle parking to | 59 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| parallel (22) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| set appropriate speed limit (1,15) | 20 |  | 35 |  |  |  |  |  |  |  |  |  |  |  |  |
| prohibit left-turns (1, 9) | 45 |  |  |  | 30 |  |  | 90 |  |  | 10 |  |  |  |  |
| change two-way roadway to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| one-way roadway (1, 23) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - intersection crashes | 26 |  |  |  |  |  |  |  |  |  | 46 |  |  |  |  |
| - mid-block crashes | 43 |  |  |  |  |  |  |  |  |  | 50 |  |  |  |  |
| prohibit right-turn-on-red at |  |  |  |  | 20 | 30 | 20 |  |  |  | 30 |  |  |  |  |
| signalized intersections (9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### ROADSIDE IMPROVEMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| remove fixed objects (1) | 30 | F:50 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I:30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| relocate fixed objects (1) | 25 | F:40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I:25 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or |  | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| ROADSIDE IMPROVEMENT (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve gore area (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| modify poles/posts with (1) | 5 | F:60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| breakaway features |  | I:30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install impact attenuators (1) | 5 | F:75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I:50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| relocate utility poles to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| increase offset from road (16) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 2 to 6 ft |  |  |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |
| - from 3 to 8 ft |  |  |  |  |  |  |  |  |  |  | 46 |  |  |  |  |  |
| - from 5 to 10 ft |  |  |  |  |  |  |  |  |  |  | 36 |  |  |  |  |  |
| flatten side-slope (29, 30) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 2:1 to 4:1 | 6 |  |  |  |  |  |  |  |  |  | 10 |  | 10 |  |  |  |
| - from 2:1 to 5:1 | 9 |  |  |  |  |  |  |  |  |  | 15 |  | 15 |  |  |  |
| - from 2:1 to 6:1 | 12 |  |  |  |  |  |  |  |  |  | 21 |  | 21 |  |  |  |
| - from 3:1 to 4:1 | 5 |  |  |  |  |  |  |  |  |  | 8 |  | 8 |  |  |  |
| - from 3:1 to 5:1 | 8 |  |  |  |  |  |  |  |  |  | 14 |  | 14 |  |  |  |
| - from 3:1 to 6:1 | 11 |  |  |  |  |  |  |  |  |  | 19 |  | 19 |  |  |  |
| - from 4:1 to 6:1 | 7 |  |  |  |  |  |  |  |  |  | 12 |  | 12 |  |  |  |
| - from 5:1 to 7:1 | 8 |  |  |  |  |  |  |  |  |  | 14 |  | 14 |  |  |  |
| install animal fencing (1, 2) | 90* | 91 |  | 61 |  |  |  |  |  |  |  |  |  |  |  |  |
| eliminate poles by burying | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| utility lines (31) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install object markers (2) | 16 | F:41 |  | 14 |  |  |  |  |  |  |  |  | 29 |  |  |  |
|  |  | I:17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * Applies to animal-related crashes only |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| ROADSIDE IMPROVEMENT (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| increase roadside clear zone |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| recovery distance (10) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - add 5 ft |  |  |  |  |  |  |  |  |  | 13 |  | 13 |  |  |  |
| - add 8 ft |  |  |  |  |  |  |  |  |  | 21 |  | 21 |  |  |  |
| - add 10 ft |  |  |  |  |  |  |  |  |  | 25 |  | 25 |  |  |  |
| - add 15 ft |  |  |  |  |  |  |  |  |  | 35 |  | 35 |  |  |  |
| - add 20 ft |  |  |  |  |  |  |  |  |  | 44 |  | 44 |  |  |  |
| #### DELINEATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install post-mounted delineators | 25 |  |  |  |  |  |  |  |  |  |  |  |  | 30 |  |
| on horizontal curve (1, 15) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install chevron alignment | 35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sign on horizontal curve (15) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install delineation at bridges (5) | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

REFERENCES FOR ESTIMATED CRASH REDUCTION FACTORS

- “Development of Accident Reduction Factors,” Kentucky Transportation Center, College of Engineering, Research Report KTC-96-13.

- “Analysis of Traffic Records: Potential Implications for Accident Reduction Factors,” 1996 International Forum on Traffic Records and Highway Information Systems, July 1996.

- Graham, J. and J. Glennon, “Manual on Identification, Analysis and Correction of High Accident Locations,” Missouri State Highway Commission, November 1975.

- “A Study of Motor Vehicle Traffic Accidents at Bridges on the Colorado State Highway System,” Colorado State Department of Highways, Planning and Research Division, June 1973.

- Lee, J., et al., “Measure the Effectiveness of Highway Safety Projects and to Improve Forecasts of Accident Reduction in Kansas,” University of Kansas, Transportation Center, February 1981.

- “Evaluation of Minor Improvements (Parts 1-6),” California Department of Public Works, Division of Highways, Traffic Department, May 1967.

- Neuman, T., “Intersection Channelization Design Guide,” National Cooperative Highway Research Program Report 279, Transportation Research Board, 1985.

- Thakkar, J., “Study of the Effect of Two-Way Left-Turns Lanes on Traffic Accidents,” Transportation Research Record 960, Transportation Research Board, 1984, pp.27-33.

- Barbaresso, J., et. al., “Selection Process for Local Highway Safety Projects,” Transportation Research Record 847, Transportation Research Board, 1982, pp.24-29.

- Zegeer C., et al., “Safety Cost-Effectiveness of Incremental Changes in Cross-Section Design- Informational Guide,” Federal Highway Administration Report No. FHWA/RD-87/094, December 1987.

- Creasely, T. and K. Agent, “Development of Accident Reduction Factors,” University of Kentucky, Report No. UKTRP-85-6, March 1985.

- “Designing Safer Roads- Practices for Resurfacing, Restoration, and Rehabilitation,” Special Report 214, Transportation Research Board, 1987, pp.256-264.

- “Selecting and Making Highway Safety Improvements: A Self-Instructional Text,” Institute of Transportation Engineers, TTC 440, 1977.

- McCoy, J., “Safety Improvement Economic Analysis,” Iowa Department of Transportation, Memo Reference Number 590, December 20, 1985.

- Wattleworth, J., et al., “Accident Reduction Factors for Use in Calculating Benefit/Cost – Florida Manual of Identification, Analysis and Correction of High Accident Locations,” University of Florida, November 1988.

- Zegeer, C. and M. Cynecki, “Selection of Cost-Effective Countermeasures for Utility Pole Accidents – User’s Manual,” Federal Highway Administration, Report No. FHWA-IP-86-9, December 1986.

- Hammer, J. and E. Tye, “Overhead Yellow-Red Flashing Beacons,” California Department of Transportation, Division of Traffic Engineering, Report No. FHWA/CA/TE-87/01, January 1987.

- Tamburri, T. and R. Smith, “The Safety Index: A Method of Evaluating and Rating Safety Benefits,” Highway Research Record 332, Highway Research Board, 1970, pp. 28-43.

- “Accident Identification & Surveillance Documentation Manual,” University of Alabama, TSM Report No. 112-88, September 1988.

- Hatton, R., “The Pavement Marking Demonstration Program – One State’s View,” Proceedings ASCE Specialty Conference, Implementing Highway Safety Improvements, pp. 149-164, March 1980.

- “Accident Reduction Factors,” New York State Department of Transportation, Traffic and Safety Division, January 1989.

- “Abilene Traffic Operations Plan,” Pinnell-Anderson-Wilshire and Associates, Inc., 1975.

- “Synthesis of Safety Research Related to Traffic Control and Roadway Elements,” Volume 1-2, Federal Highway Administration, Report Numbers FHWA-TS-82-232/233, December 1982.

- “Accident Reduction Factors – State of Kansas HES Project Evaluations,” Kansas Department of Transportation, Bureau of Traffic Engineering, June 1990.

- Benioff, B. and T. Rorabaugh, “A Study of Clearance Intervals, Flashing Operation, and Left-Turn Phasing as Traffic Signals,” Federal Highway Administration, Report Number FHWA-RD-78-46, May 1980.

- Lovell, J. and E. Hauer, “The Safety Effect of Conversion to All-Way Stop Control,” Transportation Research Record 1068, Transportation Research Board, 1986, pp. 103-107.

- Laughland, J., et. al., “Methods for Evaluating Highway Safety Improvements,” National Cooperative Highway Research Program Report 162, Transportation Research Board, 1975.

- Harwood, D., “Relationships Between Operational and Safety Considerations in Geometric Design Improvements,” Transportation Research Record 1512, Transportation Research Board, December 1995.

- Zegeer C., et. al., “Safety Effects of Cross-Section Design for Two-Lane Roads,” Federal Highway Administration Report No. FHWA/RD-87/008, October 1987.

- Zegeer, C. and F. Council, “Safety Relationships Associated with Cross-Sectional Roadway Elements,” Transportation Research Record 1512, Transportation Research Board, December 1995.

- Al-Masaeid, H. and K. Sinha, “Analysis of Accident Reduction Potentials of Pavement Markings,” Journal of Transportation Engineering, Vol. 120, No. 5, Sept.- Oct. 1994.

- “Table 3- Accident Reduction Factors” Nebraska Department of Roads (internal document).

APPENDIX H

ECONOMIC ANALYSIS:  COST UPDATES, CRASH COSTS, COMPOUND INTEREST FACTORS, AND THEIR APPLICATIONs

COST UPDATES

The countermeasure costs listed in Appendix E, as well as crash costs used in this edition of the HAL Manual, apply to the State of Missouri for the year 1999.  The city engineer or other local official who is responsible for applying the HAL Manual in future years may want to update these costs using one of the following methods:

- Adjust all costs using an annual percentage increase for each type of cost.  This would be a tedious process, but it might be necessary due to the rapidly increasing cost of fatal and injury vehicle crashes relative to other cost categories.

- Contact the TTAP office to obtain costs currently used by MoDOT in their high-hazard elimination program.

- Assume a reasonable rate of increase per year for all costs involved, such as 4 or 5 percent per year.

- Use the costs as provided in the HAL Manual, assuming all costs increased in a compatible manner, thereby having little or no effect on the results of the benefit/cost ratio computations.

CRASH COSTS

The crash costs, as stated in Chapter 1 and applied in Chapter 5, assume a 1999 basis and are:

Cost of a Fatal (F) Crash:				$3,390,000

Cost of an Injury (I) Crash:				$     44,100

Cost of a Property-Damage-Only (PDO) Crash:	$       3,220

For several reasons, it is not recommended that the cost for a fatal crash be applied directly as the amount shown above.  Fatal crashes are infrequent events, and, if the $3,390,000 cost is applied, the chance occurrence of one fatal crash at a site would overwhelmingly influence the selection process.  This could result in omitting another site for improvement, which had a larger number of serious injury crashes, but did not experience a fatal crash.  Furthermore, reliable crash reduction factors suitable for application to fatal crashes are not readily available due to the infrequency of such events and the difficulty of developing the factors.

To counteract these problems, it is assumed that fatal crashes and injury crashes are events which can each be expressed as a percentage of the total fatal and injury crashes occurring statewide on a specific classification of highway system.  The percentages for fatal crashes and injury crashes can be applied to the cost of a fatal crash and to the cost of an injury crash, respectively, to develop a crash category known as “Fatal or Injury Crash.”  The formula to describe this is:

| Cost of F+I Crashes = | (F%) (F Crash Cost) + (I%) (I Crash Cost) |
| --- | --- |
|  |  |
|  | (100%) |

For this edition of the HAL Manual, data published by the Missouri State Highway Patrol (“Missouri Traffic Crashes”) were used to compute the percentages for fatal crashes and for injury crashes on six classifications of Missouri traffic-ways.  These percentages were then applied to the cost of a fatal crash and the cost of an injury crash to yield the weighted cost of a Fatal or Injury Crash, as shown in the last column of Table H-1.

| Classification of Traffic-way | Percent Fatal Crashes | Percent Injury Crashes | Weighted Cost of Fatal or Injury Crashes, in $ |
| --- | --- | --- | --- |
| Interstate | 2.812 | 97.188 | 138,000 |
| U.S. Numbered | 3.062 | 96.948 | 147,000 |
| State Numbered | 2.835 | 97.165 | 139,000 |
| State Lettered | 3.875 | 96.125 | 174,000 |
| County Road | 2.193 | 97.805 | 117,000 |
| City Street | 0.745 | 99.255 | 69,000 |

TABLE H-1:  COST OF FATAL OR INJURY CRASHES OCCURRING ON SIX CLASSIFICATIONS OF TRAFFIC-WAY IN MISSOURI.

Since the HAL Manual is primarily intended to be used as a guide for conducting traffic safety studies in communities, the weighted cost of fatal or injury crashes on city streets ($69,000) is used for the example in Chapter 5.

COMPOUND INTEREST FACTORS

A compound interest rate of 4 percent per year is used in the HAL Manual example computations.  Rates other than 4 percent could be used, depending on local policy or on factors such as the interest rate on local bond issues.

To perform an analysis involving interest factors, it is convenient to apply factors that have already been tabulated.  The two categories of interest factors needed for most traffic safety analyses are known as the “Capital Recovery Factor” and the “Sinking Fund Factor.”  Tabulations of these factors for compound interest rates of 3%, 4%, and 5% are provided in Tables H-2, H-3, and H-4, respectively.

Examples Showing Interest Factor Applications

- Example 1:	Paint center-lines, lane lines, crosswalks, and lane use arrows on four approaches at an intersection.

$200 initial cost

$0 residual value

Service life of 1 year

Determine equivalent uniform annual cost (A) using 4% interest

A = P(A/P,4%,1) = 200(1.04) = $208 per year

- Example 2:	Install 4 regulatory and 4 warning signs at an intersection.

$720 initial cost

$50 residual value (for sign materials)

Service life of 7 years

Determine equivalent uniform annual cost (A) using 4% interest

A = P(A/P,4%,7) - F(A/F,4%,7) = 720(0.16661) - 50(0.12661)

A = 119.96 – 6.33 = $113.63 per year

- Example 3:	Install intersection lighting using two poles.

$3,200 initial cost

$800 residual value

Service life of 15 years

Determine equivalent uniform annual cost (A) using 4% interest

A = P(A/P,4%,15) – F(A/F,4%,15) = 3200(0.08994) – 800(0.04994)

#### A = 287.81 – 39.95 = $247.86 per year

- Example 4:	Determine the total equivalent uniform annual cost (A) for a set of three improvements to be made at one location.  The three improvements are the items specified in Examples 1, 2 and 3.  Use a 4% interest rate.

The three types of improvements for this location have different service lives, which means a special procedure must be followed to find the total equivalent uniform annual cost (A).

First, it is necessary to assume that when each improvement reaches the end of its service life, it will be replaced by an identical item having similar costs.  This pattern of replacing items is assumed to continue for a long time.

Next, the equivalent uniform annual cost is calculated for each type of improvement by using the costs associated with the first item in the series of identical replacements.

Finally, the total equivalent uniform annual cost is found by adding the annual costs for the first item from each of the three types of improvements.  Since the equivalent uniform annual cost has already been calculated for each improvement project, the total equivalent uniform annual cost in Example 4 is found by adding together the previous results:

A = 208 + 113.63 + 247.86 = $569.49 per year

REFERENCES FOR ECONOMIC ANALYSIS

Grant, E., W. Ireson, and R. Leavenworth, “Principles of Engineering Economy,” John Wiley & Sons, New York, New York, 8th Edition, 1990.

“A Manual on User Benefit Analysis of Highway and Bus-Transit Improvements,” American Association of State Highway and Transportation Officials, 1977.

- “Missouri Traffic Crashes,” Missouri State Highway Patrol, Department of Public Safety, published annually.

“Motor Vehicle Accident Costs,” Federal Highway Administration Technical Advisory, T-7570.1, June 30, 1988.  Attachment:  A. Bailey, “Accident Costs – Are We Using Them Correctly?”

table h-2:  interest factors – 3 percent compounded annually

table h-3:  interest factors – 4 percent compounded annually

table h-4:  interest factors – 5 percent compounded annually

Appendix I

HAL System Worksheets

The chapters in this manual refer to a number of worksheets and include examples of how to use them.  This appendix contains copies of the worksheets for use by those conducting the HAL analysis.  The worksheets may be copied freely, as needed.

CONDITION DIAGRAM			[form OSOR-3]

Safety Audit Checklists for Existing Streets

Auditor(s):  _______________________________________	Date:  _________________

Location (Reference Map included):

TRAFFIC SIGNS

Traffic signs must: 1) Fulfill a need, 2) Command attention, 3) Convey a clear, simple message, 4) Command respect of road users, and 5) Give adequate time for proper response.  When correcting problems, priority is recommended for regulatory signs (i.e. Stop, Yield, Speed Limit, Do Not Enter, and Road Closed) and for major warning signs (i.e. Stop Ahead, Yield Ahead, Turn, Curve, and Railroad Crossings).

Check

Are signs visible, both day and night, at a distance that provides response time for motorists?

Is sign visibility affected by:

	Vegetation, Dirt, Other Materials?

	Sharp Curves?

	Steep Hills?

	Other Signs?

	Poor Lighting?

	Reflectivity at Night?

Have damaged, vandalized, or missing signs been repaired or replaced?

Does the sign have a clear and simple message?

Are signing practices consistent at similar locations?

Are signs correctly positioned with respect to:

	Lateral Clearance?  (2 feet recommended)

	Height?  (7 feet to bottom of the sign recommended)

Are sign supports breakaway or yielding?

	If not, are the sign supports located to minimize exposure to traffic?

Site-specific factors may require engineering judgment.  The Manual on Uniform Traffic Control Devices (MUTCD) is the basis for all traffic control device standards.  The MUTCD and applicable state and local standards should be referenced as needed.  The necessary advance warning distance depends on several factors such as vehicle speed, site conditions, and required motorist action; consult the MUTCD for further guidance.

Safety Audit Checklists for Existing Streets

Auditor(s):  _______________________________________	Date:  _________________

Location (Reference Map included):

INTERSECTIONS

Site-specific factors often require engineering judgment.  The Manual on Uniform Traffic Control Devices (MUTCD) and applicable state and local standards should be referenced as needed for guidance as to the appropriate traffic control and sight distance for an intersection.  The signing checklist provides a more detailed examination of signing issues.

Check

Is the visibility of the intersection or any approaches limited by:

	Parked or Queued Traffic?

	Signs, Utility Poles, Fences?

	Embankments?

	Buildings?

	Vegetation?

	Other Sight Obstructions?

Has an effort been made to improve the sight distance of the intersection before installing traffic control measures?

	An engineering study is usually necessary for the placement of traffic control.

	Use of stop signs is not recommended for speed control.

Are hidden or unexpected intersections located on:

	Hills or curves?

	At the end of high-speed streets?

	Streets that do not intersect at 90?

If so, additional warning for the motorist may be necessary.

Are pedestrians (children, bicyclists, etc.) and motorists readily visible at the intersection?

| Intersection Traffic Conflict Summary [Form ITCS] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location _____________________________________________ Date_______________ 8 1=N 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observer ____________________________________________ Day _______________ Leg Number: _______ 7 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| [C = Conflict SC = Secondary Conflict] Time of Study: From: _________ To: ____________ 6 5 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Time Start  End | OPPOSING |  | FROM LEFT |  |  |  |  |  | SAME DIRECTION |  |  |  |  |  |  |  | FROM RIGHT |  |  |  |  |  | OTHER |  |
|  | Left Turn |  | Left Turn |  | Thru |  | Right Turn |  | Left Turn |  | Slow Vehicle |  | Lane Change |  | Right Turn |  | Left Turn |  | Thru |  | Right Turn |  |  |  |
|  | C | SC | C | SC | C | SC | C | SC | C | SC | C | SC | C | SC | C | SC | C | SC | C | SC | C | SC | C | SC |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUM C+SC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COMMENTS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

APPENDIX G

ESTIMATED CRASH REDUCTION FACTORS

The estimated crash reduction (CR) factors in Table G-1 are based on safety project evaluations performed by a variety of groups and agencies throughout the United States.  Due to the variability in traffic crash characteristics and countermeasure effectiveness among sites and regions, differences in CR factors for specific improvements do exist among agencies.  Whenever possible, an agency should monitor its traffic safety improvement projects and develop its own CR factors.

CR factors are required for estimating the economic benefits likely to result from feasible countermeasures.  Each CR factor indicates the percent crash reduction for a single countermeasure.

When applying CR factors, good engineering judgment and common sense must prevail.  It is essential that each CR factor be applied to only those crashes having a reasonable chance of being corrected by the associated countermeasure.

The Estimated Crash Reduction Factor table is organized according to countermeasure category and CR factor group.  The countermeasure categories are printed in capital letters in the left column, and the CR factor groups are identified by Roman numerals at the top of the table.

COUNTERMEASURE CATEGORIES

The countermeasure categories are tabled in the following sequence:

- Channelization

- Construction/Reconstruction

- Traffic Signs

- Traffic Signals

- Illumination

- Pavement Treatment

- Pavement Markings

- Regulations

- Roadside Improvement

- Delineation

Within each major countermeasure category, sub-categories are listed.  For instance, under the category “REGULATIONS” there are sub-categories such as “Regulate On-Street Parking” and “Prohibit Left Turns.”

When several countermeasures are being considered for simultaneous use to correct a crash pattern at one location, the combined effect must be calculated using the procedure in the section entitled “COUNTERMEASURE ANALYSIS” in Chapter 5.  If that procedure is not followed, the crash reduction estimate will be incorrect.

CRASH REDUCTION (CR) FACTOR GROUPS

The CR factors are grouped to provide guidance for their proper application.  The five groups listed across the top of the table are defined as follows:

GROUP I:	Contains CR factors applicable to “All” crashes.

GROUP II:	Contains CR factors applicable to crashes according to severity level, “Fatal/Injury” or “PDO”.

GROUP III:	Contains CR factors applicable to several different types of crashes, such as “Head On” or “Right Angle”.

GROUP IV:	Contains CR factors applicable to crashes that occur during “Wet Pavement” conditions.

GROUP V:	Contains CR factors applicable to crashes that occur during “Night” conditions.

GROUP VI:	Contains CR factors applicable to crashes that are train-related.

It is recommended that, for a specific countermeasure, the CR factor(s) to be applied should be selected from only one of the five groups.  For example, if the countermeasure is “PAVEMENT TREATMENTS – de-slick pavement” for a high-crash intersection, the engineer should choose the most meaningful application of CR factors from these possibilities:

- From Group I:  Apply 13% reduction to All crashes; or

- From Group III:  Apply CR factors to specific crash types, as:  10% reduction to Head On; 40% to Rear End, 10% to Right Angle, 10% to Side-Swipe; 10% to Fixed Object; 10% to Pedestrian, and 10% to Run-Off Road crashes; or

- From Group IV:  Apply 55% reduction to Wet Pavement crashes.

If CR factors are applied from more than one group for the proposed “De-slicking” countermeasure, the crash reduction may be substantially overestimated.  Of course, the ideal situation would be to have CR factors for both wet and dry pavement conditions, for each crash type, and for each level of severity.  However, CR factors are seldom available at that level of detail.

For additional access control measures, see appendix D.  Table D-3 contains information on crash reduction, in a different format, as a function of ADT.

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| #### CHANNELIZATION (see also Table D-3 in Appendix D) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| channelize intersection (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| provide left-turn lane (with | 25 |  |  |  |  |  |  | 45 |  |  |  |  |  |  |  |
| signal) (1, 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - with no left-turn phase | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - existing left-turn phase | 35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| provide left-turn lane (without | 35 |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  |
| signal) (1, 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - painted lane | 32 |  |  |  | 75 |  |  |  |  |  |  |  |  |  |  |
| - protected lane with curb or | 67 |  | 62 |  | 93 |  |  |  |  |  |  |  |  |  |  |
| raised bars |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| provide right-turn lane (1) | 25 |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |
| increase turn lane length (1) | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install two-way left-turn | 35 | 20 | 35 |  | 36 |  |  | 33 |  |  |  | 37 |  |  |  |
| lane in median (2, 8, 28) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - two-lane to three-lane | 32 | 59 |  |  | 46 |  | 46 | 46 |  |  |  |  |  |  |  |
| - four-lane to five-lane | 28 | 42 |  |  | 40 |  | 40 | 40 |  |  |  |  |  |  |  |
| add mountable median (1) | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add non-mountable median (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### CONSTRUCTION/ RECONSTRUCTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REALIGNMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| construct a more gradual | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| horizontal curve (1,12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 20 to 10 degrees | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 15 to 5 degrees | 63 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 10 to 5 degrees | 45 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1:  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| CONSTRUCTION/ RECONSTRUCTION (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REALIGNMENT (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve vertical curve (1) | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve horizontal and | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| vertical curve (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve sight distance at | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| intersection (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SEPARATING DEVICES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| close median opening (3) |  |  |  | 100 | 50 | 100 | 50 | 100 |  |  |  |  |  |  |  |
| install median barrier (1, 2, 5) | 5 | F:65 |  |  |  |  |  |  |  |  |  | 35 |  |  |  |
|  |  | I:40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - install a 1 to 12 ft. median |  | F:75 | -28* |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I:2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - install a 13 to 30 ft. median |  | F:85 | -30* |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I:5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install concrete median |  | F:90 | -10* |  |  |  |  |  |  |  |  |  |  |  |  |
| barrier (5) |  | I:10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install/improve curbing (9) |  |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |
| replace active warning | 95 |  | 88 |  |  |  |  |  |  |  |  |  |  |  |  |
| devices with bridge or tunnel (5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * A crash reduction factor preceded by a (-) sign indicates an increase should be expected for that type of crash. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| CONSTRUCTION/ RECONSTRUCTION (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PAVEMENT WIDENING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| widen pavement (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| widen shoulder (paved) (10) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - widen 2 ft. |  |  |  |  |  |  |  |  |  | 16 |  | 16 |  |  |  |
| - widen 4 ft. |  |  |  |  |  |  |  |  |  | 29 |  | 29 |  |  |  |
| - widen 6 ft. |  |  |  |  |  |  |  |  |  | 40 |  | 40 |  |  |  |
| - widen 8 ft. |  |  |  |  |  |  |  |  |  | 49 |  | 49 |  |  |  |
| widen shoulder (unpaved) (10) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - widen 2 ft. |  |  |  |  |  |  |  |  |  | 13 |  | 13 |  |  |  |
| - widen 4 ft. |  |  |  |  |  |  |  |  |  | 25 |  | 25 |  |  |  |
| - widen 6 ft. |  |  |  |  |  |  |  |  |  | 34 |  | 34 |  |  |  |
| - widen 8 ft. |  |  |  |  |  |  |  |  |  | 43 |  | 43 |  |  |  |
| pave shoulder (1) | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| stabilize shoulder (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| widen lane (10) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - add 1 ft. to both sides |  |  |  | 12 |  |  | 12 |  |  |  |  | 12 |  |  |  |
| - add 2 ft. to both sides |  |  |  | 23 |  |  | 23 |  |  |  |  | 23 |  |  |  |
| - add 3 ft. to both sides |  |  |  | 32 |  |  | 32 |  |  |  |  | 32 |  |  |  |
| - add 4 ft. to both sides |  |  |  | 40 |  |  | 40 |  |  |  |  | 40 |  |  |  |
| ADDITIONAL LANES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add passing/climbing lane (28) | 25 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add accel./decel. lane (1) | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add lanes (2) | 25 | F:39 | 27 | 53 | 32 |  | 30 |  |  |  |  | 44 |  |  |  |
|  |  | I:23 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| CONSTRUCTION/ RECONSTRUCTION (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BRIDGES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| widen bridge (general) (1, 2, 4) | 45 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 18 to 24 ft. | 68 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 20 to 24 ft. | 56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 22 to 24 ft. | 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 18 to 30 ft. | 93 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 20 to 30 ft. | 90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 22 to 30 ft. | 86 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| replace two-lane bridge (1, 2) | 45 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| repair bridge deck (1) | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| INTERSECTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| increase turning radii (1) | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve sight distance (1, 2, 9) | 30 |  |  | 10 |  | 21 | 10 | 13 |  |  | 10 |  |  |  |  |
| PEDESTRIAN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| construct pedestrian bridge | 5 |  |  |  |  |  |  |  |  |  | 90 |  |  |  |  |
| or tunnel (1, 13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install sidewalk (1) |  |  |  |  |  |  |  |  |  |  | 65 |  |  |  |  |
| DRAINAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| provide adequate drainage (1) | 20 |  |  |  |  |  |  |  |  |  |  |  | 40 |  |  |
| provide proper | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| superelevation (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FREEWAY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| construct interchange (1) | 55 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| modify entrance/exit ramp (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| construct frontage road (1) | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install glare screen (1) |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| CONSTRUCTION/ RECONSTRUCTION (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GUARDRAIL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install guardrail (1, 2) | 5 | F:65 |  |  |  |  |  |  |  |  |  | 30 |  |  |  |
|  |  | I:40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| upgrade guardrail (1, 2) | 5 | F:50 |  |  |  |  |  |  |  |  |  | 26 |  |  |  |
|  |  | I:35 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install at bridge (5) |  | F:90 | -110* |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I:45 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install along ditch (5) |  | 26 | -19* |  |  |  |  |  |  |  |  |  |  |  |  |
| install along embankment (5) |  | 42 | -47* |  |  |  |  |  |  |  |  |  |  |  |  |
| install to shield trees (5) |  | F:65 | -90* |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I:51 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install to shield fixed objects as |  | 31 | -45* |  |  |  |  |  |  |  |  |  |  |  |  |
| rocks and steel posts (5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### TRAFFIC SIGNS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WARNING SIGNS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install warning signs (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install warning signs in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| advance of intersections (1, 11) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - urban | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - rural | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install warning signs in | 30 | F:55 |  | 29 |  |  |  |  |  |  |  | 30 |  |  |  |
| advance of curves (1, 2, 11) |  | I:20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add signs at railroad |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 |
| crossings (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install school zone signs (1) | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install pavement condition |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |
| signs (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * A crash reduction factor preceded by a (-) sign indicates an increase should be expected for that type of crash. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| TRAFFIC SIGNS (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REGULATORY SIGNS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install stop sign (2-way) (1) | 35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Change to all-way stop sign from | 55 |  |  |  | 13 | 72 |  | 20 |  |  | 39 |  |  |  |  |
| two-way stop sign (1, 26) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install yield sign (1) | 45 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install lane use signs (27) | 30 |  |  |  | 10 |  | 20 |  |  |  |  |  |  |  |  |
| GUIDE SIGNS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install guide signs (1) | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install variable message | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sign (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### TRAFFIC SIGNALS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install signal (general) (1, 24) | 25 |  |  |  |  | 65 |  |  |  |  |  |  |  |  |  |
| - from two-way stop | 28 | 43 |  |  | -46* | 74 |  | -92* |  |  |  |  |  |  |  |
| - from two-way stop and add | 36 | 53 |  |  | 8 | 74 |  | -43* |  |  |  |  |  |  |  |
| left-turn lane |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIGNAL UPGRADE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| upgrade signal (1) | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install 12-inch lenses (1) | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install visors or back-plates (1) |  |  |  |  |  | 20 |  |  |  |  |  |  |  |  |  |
| install optically programmed | 15 |  |  | 20 | 10 | 10 |  | 10 |  |  |  |  |  |  |  |
| signal lenses (1, 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| upgrade pedestal mounted to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mast arm: pre-timed signal (24) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - no left-turn lane | 51 | 52 |  |  | 24 | 69 |  | 28 |  |  |  |  |  |  |  |
| - existing left-turn lane | 44 | 25 |  |  | 35 | 74 |  | 2 |  |  |  |  |  |  |  |
| - left-turn lane added | 84 | 87 |  |  | 72 | 83 |  | 87 |  |  |  |  |  |  |  |
| * A crash reduction factor preceded by a (-) sign indicates an increase should be expected for that type of crash. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| TRAFFIC SIGNALS (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIGNAL PHASING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve signal phasing (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add exclusive left-turn phase (1) | 25 |  |  |  |  |  |  | 70 |  |  |  |  |  |  |  |
| add protected/permissive | 10 |  |  |  |  |  |  | 40 |  |  |  |  |  |  |  |
| left-turn phase (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve timing (1) | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install/improve pedestrian | 25 |  |  |  |  |  |  |  |  |  | 55 |  |  |  |  |
| signal (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve yellow change interval (1) | 15 |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |
| add all-red interval (1) | 15 |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |
| interconnect signals (1, 15) | 15 | 29 |  |  | 20 | 10 |  | 38 | 36 |  | 10 |  |  |  |  |
| install traffic actuated signal (33) |  |  |  |  |  | 10 | 20 | 80 |  |  |  |  |  |  |  |
| REMOVAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| remove unwarranted signal (1, 9) | 50 |  |  |  | 90 | -30* |  | -10* |  |  | -10* |  |  |  |  |
| FLASHING BEACON |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install flashing beacon (1) | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install flashing beacon at | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| intersection (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install intersection advance | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| warning flashers (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install general advance warning | 35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| flashers (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * A crash reduction factor preceded by a (-) sign indicates an increase should be expected for that type of crash. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| TRAFFIC SIGNALS (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAILROAD CROSSINGS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| general railroad crossings (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 |
| add flashing lights at railroad |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 65 |
| crossings (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add automatic gates at |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 |
| railroad crossings (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add automatic gates and |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 |
| flashing lights (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### ILLUMINATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve street lighting (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  | 50 |  |
| install/improve lighting at | 25 |  |  |  |  |  |  |  |  |  |  |  |  | 45 |  |
| roadway segment (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install/improve lighting at | 30 |  |  |  |  |  |  |  |  |  |  |  |  | 50 |  |
| intersections (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install/improve lighting at | 25 |  |  |  |  |  |  |  |  |  |  |  |  | 50 |  |
| interchanges (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install/improve lighting at | 30 |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 60 |
| railroad crossings (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### PAVEMENT TREATMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| de-slick pavement (9, 21) | 13 |  |  | 10 | 40 | 10 | 10 |  |  | 10 | 10 | 10 | 55 |  |  |
| groove pavement (1) | 25 |  |  |  |  |  |  |  |  |  |  |  | 60 |  |  |
| resurface curve with skid- |  |  |  | 86 |  |  |  |  |  |  |  |  | 51 |  |  |
| resistant overlay (21) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| resurface (general) (1) | 25 |  |  |  |  |  |  |  |  |  |  |  | 45 |  |  |
| install rumble strips (1, 2) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| groove shoulder (1, 2) | 25 | 18 | 17 |  |  |  |  |  |  |  |  | 27 |  |  |  |
| make surface improvements | 34 |  | 39 |  |  |  |  |  |  |  |  |  |  |  |  |
| at railroad crossings (11) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| #### PAVEMENT MARKINGS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add pavement markings (32) | 13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add pavement markings at | 48 | 42 | 51 |  | 58 |  |  |  |  |  |  |  |  |  | 15 |
| railroad crossings (1, 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add reflectorized raised | 10 |  |  | 20 |  |  | 20 |  |  | 10 |  | 10 | 25 | 20 |  |
| pavement markings (1, 9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add "no passing" striping (1) |  |  |  | 40 |  |  | 40 |  |  |  |  |  |  |  |  |
| add centerline markings (1) | 35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add edgeline markings (1, 20) | 15 | 15 | 8 |  |  |  |  |  |  |  |  | 30 |  |  |  |
| add/improve pedestrian |  |  |  |  |  |  |  |  |  |  | 25 |  |  |  |  |
| crosswalk (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add wider markings (1) |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 |  |
| #### REGULATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| prohibit on-street parking (1, 9) | 35 |  |  |  | 10 | 10 | 30 |  |  | 40 | 30 |  |  |  |  |
| change angle parking to | 59 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| parallel (22) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| set appropriate speed limit (1,15) | 20 |  | 35 |  |  |  |  |  |  |  |  |  |  |  |  |
| prohibit left-turns (1, 9) | 45 |  |  |  | 30 |  |  | 90 |  |  | 10 |  |  |  |  |
| change two-way roadway to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| one-way roadway (1, 23) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - intersection crashes | 26 |  |  |  |  |  |  |  |  |  | 46 |  |  |  |  |
| - mid-block crashes | 43 |  |  |  |  |  |  |  |  |  | 50 |  |  |  |  |
| prohibit right-turn-on-red at |  |  |  |  | 20 | 30 | 20 |  |  |  | 30 |  |  |  |  |
| signalized intersections (9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### ROADSIDE IMPROVEMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| remove fixed objects (1) | 30 | F:50 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I:30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| relocate fixed objects (1) | 25 | F:40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I:25 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or |  | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| ROADSIDE IMPROVEMENT (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| improve gore area (1) | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| modify poles/posts with (1) | 5 | F:60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| breakaway features |  | I:30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install impact attenuators (1) | 5 | F:75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I:50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| relocate utility poles to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| increase offset from road (16) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 2 to 6 ft |  |  |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |
| - from 3 to 8 ft |  |  |  |  |  |  |  |  |  |  | 46 |  |  |  |  |  |
| - from 5 to 10 ft |  |  |  |  |  |  |  |  |  |  | 36 |  |  |  |  |  |
| flatten side-slope (29, 30) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - from 2:1 to 4:1 | 6 |  |  |  |  |  |  |  |  |  | 10 |  | 10 |  |  |  |
| - from 2:1 to 5:1 | 9 |  |  |  |  |  |  |  |  |  | 15 |  | 15 |  |  |  |
| - from 2:1 to 6:1 | 12 |  |  |  |  |  |  |  |  |  | 21 |  | 21 |  |  |  |
| - from 3:1 to 4:1 | 5 |  |  |  |  |  |  |  |  |  | 8 |  | 8 |  |  |  |
| - from 3:1 to 5:1 | 8 |  |  |  |  |  |  |  |  |  | 14 |  | 14 |  |  |  |
| - from 3:1 to 6:1 | 11 |  |  |  |  |  |  |  |  |  | 19 |  | 19 |  |  |  |
| - from 4:1 to 6:1 | 7 |  |  |  |  |  |  |  |  |  | 12 |  | 12 |  |  |  |
| - from 5:1 to 7:1 | 8 |  |  |  |  |  |  |  |  |  | 14 |  | 14 |  |  |  |
| install animal fencing (1, 2) | 90* | 91 |  | 61 |  |  |  |  |  |  |  |  |  |  |  |  |
| eliminate poles by burying | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| utility lines (31) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install object markers (2) | 16 | F:41 |  | 14 |  |  |  |  |  |  |  |  | 29 |  |  |  |
|  |  | I:17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * Applies to animal-related crashes only |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

| COUNTERMEASURE | I | II |  | III |  |  |  |  |  |  |  |  | IV | V | VI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All | Fatal or | PDO | Head | Rear | Right | Side- | Left | Right | Fixed | Pedes- | Run- | Wet | Night | Train- |
|  |  | Injury |  | On | End | Angle | Swipe | Turn | Turn | Object | trian | Off | Pave- |  | Related |
|  |  |  |  |  |  |  |  |  |  |  |  | Road | ment |  |  |
| ROADSIDE IMPROVEMENT (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| increase roadside clear zone |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| recovery distance (10) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - add 5 ft |  |  |  |  |  |  |  |  |  | 13 |  | 13 |  |  |  |
| - add 8 ft |  |  |  |  |  |  |  |  |  | 21 |  | 21 |  |  |  |
| - add 10 ft |  |  |  |  |  |  |  |  |  | 25 |  | 25 |  |  |  |
| - add 15 ft |  |  |  |  |  |  |  |  |  | 35 |  | 35 |  |  |  |
| - add 20 ft |  |  |  |  |  |  |  |  |  | 44 |  | 44 |  |  |  |
| #### DELINEATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install post-mounted delineators | 25 |  |  |  |  |  |  |  |  |  |  |  |  | 30 |  |
| on horizontal curve (1, 15) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install chevron alignment | 35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sign on horizontal curve (15) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| install delineation at bridges (5) | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table g-1 (cont’d):  estimated crash reduction factors (%)

REFERENCES FOR ESTIMATED CRASH REDUCTION FACTORS

- “Development of Accident Reduction Factors,” Kentucky Transportation Center, College of Engineering, Research Report KTC-96-13.

- “Analysis of Traffic Records: Potential Implications for Accident Reduction Factors,” 1996 International Forum on Traffic Records and Highway Information Systems, July 1996.

- Graham, J. and J. Glennon, “Manual on Identification, Analysis and Correction of High Accident Locations,” Missouri State Highway Commission, November 1975.

- “A Study of Motor Vehicle Traffic Accidents at Bridges on the Colorado State Highway System,” Colorado State Department of Highways, Planning and Research Division, June 1973.

- Lee, J., et al., “Measure the Effectiveness of Highway Safety Projects and to Improve Forecasts of Accident Reduction in Kansas,” University of Kansas, Transportation Center, February 1981.

- “Evaluation of Minor Improvements (Parts 1-6),” California Department of Public Works, Division of Highways, Traffic Department, May 1967.

- Neuman, T., “Intersection Channelization Design Guide,” National Cooperative Highway Research Program Report 279, Transportation Research Board, 1985.

- Thakkar, J., “Study of the Effect of Two-Way Left-Turns Lanes on Traffic Accidents,” Transportation Research Record 960, Transportation Research Board, 1984, pp.27-33.

- Barbaresso, J., et. al., “Selection Process for Local Highway Safety Projects,” Transportation Research Record 847, Transportation Research Board, 1982, pp.24-29.

- Zegeer C., et al., “Safety Cost-Effectiveness of Incremental Changes in Cross-Section Design- Informational Guide,” Federal Highway Administration Report No. FHWA/RD-87/094, December 1987.

- Creasely, T. and K. Agent, “Development of Accident Reduction Factors,” University of Kentucky, Report No. UKTRP-85-6, March 1985.

- “Designing Safer Roads- Practices for Resurfacing, Restoration, and Rehabilitation,” Special Report 214, Transportation Research Board, 1987, pp.256-264.

- “Selecting and Making Highway Safety Improvements: A Self-Instructional Text,” Institute of Transportation Engineers, TTC 440, 1977.

- McCoy, J., “Safety Improvement Economic Analysis,” Iowa Department of Transportation, Memo Reference Number 590, December 20, 1985.

- Wattleworth, J., et al., “Accident Reduction Factors for Use in Calculating Benefit/Cost – Florida Manual of Identification, Analysis and Correction of High Accident Locations,” University of Florida, November 1988.

- Zegeer, C. and M. Cynecki, “Selection of Cost-Effective Countermeasures for Utility Pole Accidents – User’s Manual,” Federal Highway Administration, Report No. FHWA-IP-86-9, December 1986.

- Hammer, J. and E. Tye, “Overhead Yellow-Red Flashing Beacons,” California Department of Transportation, Division of Traffic Engineering, Report No. FHWA/CA/TE-87/01, January 1987.

- Tamburri, T. and R. Smith, “The Safety Index: A Method of Evaluating and Rating Safety Benefits,” Highway Research Record 332, Highway Research Board, 1970, pp. 28-43.

- “Accident Identification & Surveillance Documentation Manual,” University of Alabama, TSM Report No. 112-88, September 1988.

- Hatton, R., “The Pavement Marking Demonstration Program – One State’s View,” Proceedings ASCE Specialty Conference, Implementing Highway Safety Improvements, pp. 149-164, March 1980.

- “Accident Reduction Factors,” New York State Department of Transportation, Traffic and Safety Division, January 1989.

- “Abilene Traffic Operations Plan,” Pinnell-Anderson-Wilshire and Associates, Inc., 1975.

- “Synthesis of Safety Research Related to Traffic Control and Roadway Elements,” Volume 1-2, Federal Highway Administration, Report Numbers FHWA-TS-82-232/233, December 1982.

- “Accident Reduction Factors – State of Kansas HES Project Evaluations,” Kansas Department of Transportation, Bureau of Traffic Engineering, June 1990.

- Benioff, B. and T. Rorabaugh, “A Study of Clearance Intervals, Flashing Operation, and Left-Turn Phasing as Traffic Signals,” Federal Highway Administration, Report Number FHWA-RD-78-46, May 1980.

- Lovell, J. and E. Hauer, “The Safety Effect of Conversion to All-Way Stop Control,” Transportation Research Record 1068, Transportation Research Board, 1986, pp. 103-107.

- Laughland, J., et. al., “Methods for Evaluating Highway Safety Improvements,” National Cooperative Highway Research Program Report 162, Transportation Research Board, 1975.

- Harwood, D., “Relationships Between Operational and Safety Considerations in Geometric Design Improvements,” Transportation Research Record 1512, Transportation Research Board, December 1995.

- Zegeer C., et. al., “Safety Effects of Cross-Section Design for Two-Lane Roads,” Federal Highway Administration Report No. FHWA/RD-87/008, October 1987.

- Zegeer, C. and F. Council, “Safety Relationships Associated with Cross-Sectional Roadway Elements,” Transportation Research Record 1512, Transportation Research Board, December 1995.

- Al-Masaeid, H. and K. Sinha, “Analysis of Accident Reduction Potentials of Pavement Markings,” Journal of Transportation Engineering, Vol. 120, No. 5, Sept.- Oct. 1994.

- “Table 3- Accident Reduction Factors” Nebraska Department of Roads (internal document).

APPENDIX K

contact information

Many organizations are listed in this manual as resources for local agencies needing assistance with the HAL program.  Addresses and phone numbers for these offices are listed in the tables below.

| MoDOT OFFICES |  |  |  |  |
| --- | --- | --- | --- | --- |
| ABBREV. | FULL NAME | ADMINISTRATING AGENCY AND/OR ADDRESS | PHONE # | FAX # |
| --- | MoDOT Web Page | www.modot.state.mo.us | --- | --- |
| Dist. 1 | Northwest Area | 3602 North Belt Highway P.O. Box 287 St. Joseph, MO 64502 | (816) 387-2350 (888) ASK-MODOT | (816) 387-2359 |
| Dist. 10 | Southeast Area | 201 North Main Street P.O. Box 160 Sikeston, MO 63801 | (573) 472-5333 (888) ASK-MODOT | (573) 472-5342 |
| Dist. 2 | North Central Area | US Route 63 P.O. Box 8 Macon, MO 63552 | (660) 385-3176 (888) ASK-MODOT | (660) 385-4195 |
| Dist. 3 | Northeast Area | 1711 S. Route 61 P.O. Box 1067 Hannibal, MO 63401 | (573) 248-2490 (888) ASK-MODOT | (573) 248-2469 |
| Dist. 4 | Kansas City Area | 5117 East 31st Street Kansas City, MO 64128 | (816) 889-3350 (888) ASK-MODOT | (816) 889-3369 |
| Dist. 5 | Central Area | 1511 Missouri Blvd. P.O. Box 718 Jefferson City, MO 65102 | (573) 751-3322 (888) ASK-MODOT | (573) 527-6891 |
| Dist. 6 | St. Louis Area | 1590 Woodlake Drive Chesterfield, MO 63017 | (314) 340-4100 (888) ASK-MODOT | (314) 340-4119 |
| Dist. 7 | Southwest Area | 3901 East 32nd Street P.O. Box 1445 Joplin, MO 64802 | (417) 629-3300 (888) ASK-MODOT | (417) 629-3140 |
| Dist. 8 | Springfield Area | 3025 East Kearney Street P.O. Box 868 Springfield, MO 65801 | (417) 895-7600 (888) ASK-MODOT | (417) 895-7711 |

| MoDOT OFFICES (cont’d) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Dist. 9 | South Central Area | 910 Springfield Road P.O. Box 220 Willow Springs, MO 65793 | (417) 469-3134 (888) ASK-MODOT | (417) 469-4555 |
| TTAP | Technology Transfer Assistance Program | MoDOT Research Development and Technology Division P.O. Box 270 Jefferson City, MO 65102 | (573) 751-3002 | (573) 526-4337 |
| TEAP | Traffic Engineering Assistance Program | MoDOT Traffic Division P.O. Box 270 Jefferson City, MO 65102 | (573) 526-0117 | (573) 526-0120 |

| ### OTHER OFFICES |  |  |  |
| --- | --- | --- | --- |
| NAME | ADDRESS | PHONE # | FAX # / WEB ADDRESS |
| Missouri State Highway Patrol | Dept. of Public Safety Missouri State Highway Patrol 1510 East Elm St. P.O. Box 568 Jefferson City, MO 65102-0568 | (573) 751-3313 | (573) 751-9419 |
| Missouri State Highway Patrol, Traffic Div. | Dept. of Public Safety Missouri State Highway Patrol 1510 East Elm St. P.O. Box 568 Jefferson City, MO 65102-0568 | (573) 751-3313 | (573) 751-9419 |
| Missouri Division of Highway Safety | 1719 Southridge Dr. P.O. Box 104808 Jefferson City, MO 65110 | (573) 751-5407 | (573) 634-5977 |
| National Safety Council | 425 North Michigan Avenue Chicago, IL 60611 | (630) 775-2056 or (800) 621-7619 | www.nsc.org |