MODIFIED SHOULDER CONSTRUCTION - ASPHALT OPTION

1.0  Alternate Construction Method.  For the asphalt option, the Modified Shoulder may be constructed monolithic with the roadway overlay in accordance with Sec 401 and these provisions; or the shoulder may be constructed separately from the roadway overlay.

2.0  Construction Requirements.  For the monolithic option, the asphalt paver shall be capable of placing the shoulder portion elevated above the roadway portion to account for the additional roll-down necessary to achieve the required compaction in both the roadway and shoulder.

2.1  Regardless of the method of construction, the compacted asphalt mixture in the Modified Shoulder shall have a minimum density of 90 percent of the theoretical maximum specific gravity.

2.2  For the monolithic option, the asphalt mix type specified for the roadway overlay shall be used in the Modified Shoulder portion at no additional cost to the commission.

3.0  Method of Measurement.  Regardless of the method of construction, the Modified Shoulder shall be measured and paid per square yard.

3.1  If the Modified Shoulder is constructed monolithic with the roadway overlay, the theoretical quantity of asphalt mixture necessary to construct the Modified Shoulder, measured in tons, will be deducted from the total tonnage measured for the combination of overlay and Modified Shoulder construction.  The quantity of asphalt mixture in the outside shoulder taper will be deducted from the total tonnage as well.

4.0  Basis of Payment.  No additional payment will be made for compliance with this provision.