FY2011 Contract Leveling Course Guidelines

- These guidelines supercede conflicts with EPG 402 Bituminous Surface Leveling.

- Plans and job special provisions are to be prepared and submitted by the districts.

- Plan templates are located in ProjectWise in the design seed files:

CLC Title.dgn

CLC Typical.dgn

CLC Quantities.dgn

CLC Transitions.dgn

CLC Traffic Control 1.dgn

CLC Traffic Control 2.dgn

CLC Traffic Control 3.dgn

- The entrance transition template is an example of how entrance details can be handled.  Districts have the flexibility to handle the entrances transitions however they see appropriate.

- All projects will use 1” thickness.

- Include an appropriate amount of additional asphalt to account for surface irregularities.  Irregularities can be estimated in various ways including field measurements of the pavement width, ruts and entrances dimensions, ARAN data and input from district construction staff.

- All striping will be done by district forces.

- 1” CLC projects should not include rumblestrips.

- Recommended Completion Dates:  	D1, D2, D3 and northern D4 – November 1, 2011

Southern D4, D5 thru D10 – December 1, 2011

- Use the following bid items:

402-05.20 Bituminous Pavement Mixture PG64-22

407-10.05 Tack Coat

616-10.05 Construction Signs

616-11.20 Installing “Drive Smart” Signs

109-10.36 Furnishing “Drive Smart” Signs (non-contractual)

620-80.65 Temporary Raised Pavement Marker, Type 2

622-40.10 Modified Cold Milling (Depth Transitions)

Note:  Do not include Mobilization

- For contract level course projects that use the Time for Completion of Work JSP, the general rule of thumb for calendar days should be two (2) calendar days for every one (1) working day with a minimum of 30 calendar days unless the district has justification for a tighter time frame due to impacts to the traveling public.

- The unit bid price for your applicable area for Bituminous Pavement Mixture PG64-22 is obtained using the most recent history available from Bid Tabs Pro.  If you have a large project or uncertainty regarding a price to use, please contact Natalie Roark (573-751-3726) Estimate and Review Engineer.