URBAN DESIGN CRITERIA - RESURFACING,

RESTORATION & REHABILITATION (3R) PROJECTS

Min. Lane Widths

Through lanes (1)

Curbed	10 ft.	3.0 m

Uncurbed	11 ft.	3.3 m

Turning lanes	10 ft.	3.0 m

Shoulder Widths

Curbed lanes	No Minimum	No Minimum

Uncurbed lanes

ADT

>1700	6 ft.	1.8 m

400--1700	3 ft.	0.9 m

<400	2 ft.	0.6 m

Max. Horiz. Curve	Existing (degree) (2)	Existing (radius - m)

Vertical Alignment

Sags (K value)	Existing (2)	Existing (2)

Stopping Sight Distance	Existing (2)	Existing (2)

Max. Grade (percent)	Existing	Existing

Cross Slope (Maximum 0.02 ft./ft.)	Existing	Existing

Superelevation		Existing (ft./ft.)	Existing (m/m)

Horizontal Clearance	(3)	(3)

Vertical Clearance	14 ft.	4.3 m

Fill Slopes (4)		Existing	Existing

Bridges (5)

(1)	Desirable minimum is 11 ft. (3.3 m).  Increase to 12 ft. (3.6 m) on designated truck routes or other routes where trucks make up 10 percent or more of average daily traffic.

(2)	Improvement is considered at high-accident locations.  Use signing when sight distance is less than established criteria for new construction.

(3)	Minimum setback should be behind paved shoulder or 2 ft. (0.6 m) behind the curb.

(4)	See 128 3R Design Criteria (Rural)

(5)	See 128 3R Design Criteria (Rural)