Job Number: __________________	County:  _____________________

Route:  _________ Over______________________

Bridge Survey Submittals (EPG 747.1.2)

___	Bridge Survey Report (See EPG 750.7.4.3 for small drainage areas)

___	Plan Sheet(s)

___	Typical Section Sheet(s)

___	Profile Sheet(s)

___	Valley Section Sheet(s) – (Stream Crossing Only)

___	Cross Section Sheet(s)

___	Soil Survey/Preliminary Geotechnical Report

___	GEO File(s) – (Stream Crossing Only)

___	GEOPAK to HEC-RAS Converter Spreadsheet(s) – (Stream Crossing Only)

___	.tin File(s)

___	GEOPAK Cross Section File(s)

___	GEOPAK Cross Section Sheets File(s)

___	Structural Rehabilitation Checklist – Signed – (Widening/Rehab. Only)

___	Bridge Survey Checklist - Signed

Common Data (All Bridge Survey Sheets)

1. Title Block (EPG 747.1.3.1.1)

___	Name of Crossing

___	Section, Township and Range

___	Route

___	County

___	Survey Made By

___	Survey Date

___	Prepared By

___	Checked By

___	Project Number

___	Station

___	Job Number

___	Bridge Number

2. Other Information (EPG 747.1.3.1.2)

___	Sheet Numbering and Naming

Plan Sheet

1. General Information for all Plan Sheets (EPG 747.1.3.2.1)

___	North Arrow

___	The Note “PLAN”– Top Center of Sheet

___	Scale

___	Graphic Scale

___	Show stations from left to right

___	Location of existing and proposed roads with stationing, labeling, bearings and dimensions

___	Horizontal Curve Data

___	Location of existing bridge with general dimensions and stationing

___	Location of proposed bridge

___	Small four section index map showing approximate course of streams and location of roads and bridges

___	Location, elevation and description of two permanent bench marks

___	State Plane Coordinate System (NAD83)

___	Vertical Datum (NAVD88) Information

___	Zone Information (East, Central, West)

___	Projection Factor

___	Fences, buildings and other improvements

___	Drainage Features

___	Temporary Bypasses

___	All utilities overhead and underground – note utilities to be placed on bridge, if any

___	Location, size and flow line elevations of stormwater drainage structures

___	Existing and proposed right of way and easements

___	Section, township and range with section lines

___	Contour Lines

___	Tie Station

2. Additional Information for Stream Crossings (EPG 747.1.3.2.2)

___	Roadway Design Frequency

___	Plot course of stream sufficiently to show extent of flood conditions – 1000’ minimum upstream and downstream of crossing

___	Contours extending at least 1,000 ft. up and down the valley from the proposed crossing

___	Meander of stream should be accurately surveyed and plotted

___	Direction of Flow

___	Channel Changes - Provide Survey Details and Profile

___	Locate and describe any features above and below the crossing which could affect the course of the stream or the layout of the bridge

___	Stream name: State whether it is a river, creek, drainage ditch, ravine, dry run, etc.

___	Location of valley sections, with stationing

___	Location of typical channel sections, with stationing

___	Areas of bank deterioration or erosion

3. Additional Information for Grade Separations (EPG 747.1.3.2.3)

___	Stationing along routes under bridge

___	Label routes under bridge – NB and SB lanes, Ramps, Outer Roads, etc.

___	Roadway Drainage Features

___	Ramp Details – Stationing, Alignment, Tie

___	Outer Roadway Details– Stationing, Alignment, Tie

4. Additional Information for Railroad Crossings (EPG 747.1.3.2.4)

___	Railroad Stationing/Mile Post

___	Location of Cross Sections

___	Horizontal Curve Data

5. Additional Information for Retaining Walls (EPG 747.1.3.2.5)

___	Proposed location of retaining wall

___	Tie Points and Offset Dimensions

___	Proposed Horizontal Alignment

___	Existing and Proposed Roadway Drainage Features

Typical Section Sheet

1. Typical Sections (Roadways, Ramps, Bypasses) (EPG 747.1.3.3.1)

___	Section Title and Station Limits

___	Profile Grade Location

___	Cross Slopes/S.E.

___	Lane, Shoulder and Sidewalk Widths

___	Median Width

___	Side Slopes

___	Clear Zone Width

___	Guardrail/Barrier/Fence Location

2. Design Traffic (Construction Year and Design Year) (EPG 747.1.3.3.2)

___	AADT

___	% Trucks

___	Design Speed

___	Traffic Handling and/or Staging

3. Other Information (EPG 747.1.3.2.3)

___	Control Points and Reference Points

___	Survey Datum

___	S.E. Transition Data Table

___	Datum Shift Information

Profile Sheets

1. General Information for all Profile Sheets (EPG 747.1.3.4.1)

___	Three Line Existing Ground Profile

___	Three Line Profile Key

___	Scale

___	Proposed profile grade line including vertical curve data

___	Grades

___	Elevation and Stationing grid labels

___	Bridge Widening Profiles (widening only)

2.  Contracted Profile (EPG 747.1.3.4.2)

___	Scale

___	Profiles

3. Additional Information for Stream Crossings/Culverts (EPG 747.1.3.4.3)

___	Extreme High Water Elevation

___	Ordinary High Water Elevation

___	Low Water Elevation

4. Additional Information for Grade Separations (EPG 747.1.3.4.4)

___	Over-Route Vertical Profiles

___	Under-Route Vertical Profiles

5. Additional Information for Railroads (EPG 747.1.3.4.5)

___	Over-Route Vertical Profiles

___	Rail Survey

6. Additional Information for Retaining Walls (EPG 747.1.3.4.6)

___	Location of Wall

___	Existing and proposed ground line profile along wall locations

Valley/Channel Sections Sheets

1. Information for Valley/Channel Section Sheets (EPG 747.3.5.1)

___	Scale

___	Valley section under nearby existing structure(s)

___	Extreme High Water Elevation

___	Distance Upstream or Downstream

___	Streambed Profile

Cross Section Sheet(s)

1. General Information for all Cross Section Sheets (EPG 747.1.3.6.1)

___	Scale

___	Legend

___	Cross Section Sheets File(s)

___	Existing and Proposed ground lines

2. Additional Information for Grade Separations (EPG 747.1.3.6.2)

___	Profile Sheet Substitute

___	Drainage Features

3. Additional Information for Railroad Crossings (EPG 747.1.3.6.3)

___	Cross-sections (three) perpendicular to track

___	Rail Survey Profile Substitution

4. Additional Information for Retaining Walls (EPG 747.1.3.6.4)

___	Cross sections every 10’

Aerial Image Overlay

1. Information for Aerial Image Overlay (EPG 747.1.4.1)

___	Aerial Image Overlay (Optional)

Prepared By: ______________________________________

Title: ____________________________________________

Date: ____________________________

Checked By: ______________________________________

Title: ____________________________________________

Date: ____________________________